|  | Technical Memorandum |  |
| --- | --- | --- |
|  | Date: | DRAFT — 08/26/2026 |
|  | Project: | Mount St. Helens Project Cowlitz River Levee Systems |
|  | Subject: | Regulated Flow Frequency, Lower Cowlitz River |
|  | From: | Hydrology and River and Engineering Section, Portland District |

# 1. Purpose

This memorandum documents the development of a regulated peak flow frequency curve for the lower Cowlitz valley in Cowlitz County, Washington to inform levee assessments and floodplain mapping. Unregulated curves are developed for the instantaneous peak, 1-, 3-, and 5-day durations (Section 4) and used to develop regulated flow frequency curves for locations along the lower Cowlitz River reflecting current Water Control Manual flood risk management operations (Section 5).

# 2. Basin Description and Project Operation

The Cowlitz River drains the southern Washington Cascades and enters the Columbia River near Longview. The Castle Rock gage (USGS 14243000) sits in the lower valley, downstream of the Toutle River confluence, with a peak-flow record spanning water years 1927 through 2026. The total Cowlitz basin area is 2,480 square miles, of which 1,170 square miles (roughly half) are regulated by Mossyrock Dam. Mossyrock Dam, completed in 1968, provides the only usable flood storage in the basin. Just downstream of Mossyrock is Mayfield Dam, which acts as a re-regulating dam that smooths power peaking releases from Mossyrock providing no meaningful flood regulation of its own. Cowlitz Falls Dam above Mossyrock and the Sediment Retention Structure on the Toutle River are both run-of-river.

Figure 2-1. Cowlitz River basin, showing the Castle Rock gage, the Mossyrock and Mayfield projects, and the Toutle and Tilton tributaries.

## 2.1 Previous Studies

The previously adopted unregulated and regulated frequency curves at Castle Rock come from the 2009 Cowlitz Hydrology Restudy which predates Bulletin 17C, does not include the most recent 17 years of record, and used a limited set of synthetic hydrographs to inform the upper portion of the regulated flow frequency curve. Section 6 compares the current study results against the 2009 study.

# 3. Data Used

Table 3-1 lists the datasets used and the purpose each served.

| Dataset or model | Period used | Purpose |
| --- | --- | --- |
| Mossyrock pool elevation, hourly | Nov 1974 – May 2026 | Reservoir storage change at hourly resolution; basis of the hourly unregulated peak estimates (5.2) |
| Mossyrock storage telemetry, hourly | Regulated era | Data quality screening |
| Mossyrock pool elevation, daily | Oct 1973 – present | Storage change for developing unregulated flows (5.3, 5.4); observed starting pool for the calibration runs (5.1) |
| Mossyrock elevation-storage rating table | 2014 Water Control Manual | Relates pool elevation to reservoir storage |
| Castle Rock streamflow, hourly (USGS 14243000) | Regulated era | Regulated flow at the study location |
| Castle Rock streamflow, daily and annual peak flows (USGS 14243000) | WY1927 – present | Pre-regulation data used in unregulated flow-frequency; regulated daily flow later adjusted to unregulated flows and used in the frequency analysis. |
| Mayfield outflow, hourly (USGS 14238000) | Regulated era | Upstream boundary for routing the Mossyrock regulation signal to Castle Rock |
| Coweeman at Kelso streamflow, hourly (Washington Department of Ecology 26C075) | WY 2009 - 2016 | Lag between Coweeman River peak flows and Cowlitz River peak flows (7.3) |
| Coweeman at Kelso annual peak flows (USGS 14245000) | WY 1950 - 1996 | Peak unregulated flow correlation with Cowlitz at Castle Rock (7.2) |
| Prism Annual Precipitation | WY1950 - 2020 | Basis for drainage basin ratio (7.2) |
| Mossyrock inflow and Castle Rock local flow | WY 1929 – 2026 | Simulation inflows, back-calculated and volume-corrected |

Table 3-1. Datasets used, and their purpose.

# 4. Unregulated Flow Frequency at Castle Rock

Unregulated flow at Castle Rock is the flow that would have occurred without regulation from Mossyrock reservoir. For water years 1927–1968 the gage record reflects unregulated flows (Section 4.4). For the post 1968 regulated period the reconstruction uses an hourly holdout, where the hourly elevation record is reliable (Section 4.1), and otherwise uses a peak estimated by regression from the daily reservoir record (Section 4.2). The 3- and 5-day durations, needing only daily resolution, come from a daily mass balance (Section 4.3). The basis for calculating unregulated flows from a regulated record relies on adding back the Mossyrock reservoir holdout (reservoir storage change expressed as a flow rate) to the observed gage record downstream of the dam. A positive holdout indicates the reservoir filling.

## 4.1 Hourly Unregulated Record

The cleaned hourly pool elevation was converted to storage using the 2014 Water Control Manual rating curve and differenced to obtain an hourly holdout smoothed with a 3-hour centered moving average and restricted to the October–March flood season. Days with fewer than four valid storage readings were omitted, since they cannot be relied on to capture the annual peak.

The reservoir effect was translated to Castle Rock using the SSARR routing parameters in Table 4-1 from the calibrated Cowlitz CWMS model. Mayfield outflow plus the holdout, and Mayfield outflow alone, were each routed to Castle Rock. The difference between the two routed hydrographs is the routed reservoir effect. This time-series was added to observed Castle Rock flow to give the hourly unregulated estimate. The annual unregulated peak was extracted from this dataset.

| Reach | KTS | n | Phases |
| --- | --- | --- | --- |
| Mayfield outflow to Cowlitz River above Toutle River | 5 | 0.1 | 5 |
| Cowlitz River above Toutle River to Cowlitz River below Toutle River | 1 | 0.2 | 1 |
| Cowlitz River below Toutle River to Castle Rock | 1 | 0.2 | 5 |

Table 4-1. SSARR routing parameters, from the calibrated lower Cowlitz routing model.

Twenty-three water years produced holdout-based peaks (1992, 1997, 1999, 2003, 2004, and 2008-2026) with earlier hourly pool elevation records too unstable to use.

## 4.2 Peak Estimation by Storage-Change Regression

For years without a usable hourly record (roughly a third of the regulated era) the unregulated peak must be estimated using the daily reservoir elevation, requiring a different approach. While daily data is less useful than hourly data, it still provided good predictive value in unregulated peak estimation. For the years with hourly data, the increase in peak flow from regulated to unregulated was regressed against various predictor candidates. The predictor candidates were  the largest 1- to 4-day increase in Mossyrock storage near the event (within 7 days of the regulated peak). This is preferred over regressing the unregulated peak directly, since the change in storage at Mossyrock is directly related to the increase in flow from regulated to unregulated conditions. The predictor (daily reservoir data) and the regulated peak are both available for every regulated year, and because the adjustment is a fraction of the flow being reconstructed, a given percentage error translates to a much smaller error on the resulting unregulated peak.

| Storage-change window (candidate predictor variables) | Slope | Intercept (cfs) | R² | Standard error (cfs) |
| --- | --- | --- | --- | --- |
| 1-day | -0.673 | -7,751 | 0.759 | 7,565 |
| 2-day (adopted) | -0.869 | -3,836 | 0.871 | 5,528 |
| 3-day | -0.983 | -4,537 | 0.853 | 5,896 |
| 4-day | -1.140 | -4,770 | 0.836 | 6,239 |

Table 4-2. Candidate regressions of the regulated-minus-unregulated peak difference on Mossyrock storage change, fit to 17 water years.

The two-day window was adopted (R² = 0.871, standard error ≈ 5,530 cfs, n = 17). Pairs whose regulated and unregulated peaks fell more than 72 hours apart were excluded as different storms. Thirty water years were filled by regression. For 1974–1991, where the hourly record does not reach, the regulated peak came from the USGS annual peak record and the peak date centered the storage-change window.

Figure 4-1. Adopted two-day storage-change regression. Points are labeled by water year.

## 4.3 Durations from the Daily Mass Balance

Daily unregulated flow is the observed daily flow plus the daily change in Mossyrock storage as a flow rate. No routing is applied since Mayfield-to-Castle Rock travel time is under a day. The 1-day duration flow is calculated from an hourly record, when available, since the true 1-day maximum value often spans midnight. When no hourly data is available the 1-Day duration is calculated from daily values. The 3-Day and 5-Day durations were calculated from daily data.

## 4.4 Pre-Regulation Period

The observed gage record before December 1968 is unregulated. Peaks for water years 1927–1968 are the USGS observed instantaneous annual peaks (42 water years), and the 1-, 3-, and 5-day values are rolling maxima from the USGS daily record. Water year 1927 carries a peak only.

## 4.5 Record Extension and Historical Floods

Correlation-based extension from a nearby gage was considered, but no suitable gage was found. Nearby long-record gages either do not predate Castle Rock by much, are themselves regulated over the period that would be used, or do not correlate strongly enough to justify the added estimation error. With roughly a century of systematic record already available, the potential gain was small. A search of USGS qualification codes and published USGS/NWS accounts of major southwest Washington floods turned up no pre-1927 event at Castle Rock that could be quantified or reliably bounded. The 2009 study contained some historic estimates citing newspaper clippings, but sources were not traceable and newspaper clippings found during this study were roughly defined in terms of multiples of recorded events. No historical events or Bulletin 17C perception thresholds were used.

## 4.6 Assembled Record

For water years 1927–1968 the record is the observed gage record. For 1974–2026 the peak is the hourly holdout where available and a regression estimate otherwise. The 1-day value follows the same split based on hourly data availability, and the 3- and 5-day values always come from the daily mass balance (Table 4-3, Appendix A). Coverage runs from WY1927 through WY2026 except for WY1969–1973, which have no daily Mossyrock elevation record. Every included water year has a peak.

| Duration | Values | Sources |
| --- | --- | --- |
| Peak | 95 | 42 pre-regulation USGS peaks; 30 storage-change regression; 23 hourly holdout |
| 1-Day | 94 | 41 pre-regulation USGS daily; 30 daily mass balance; 23 hourly holdout |
| 3-Day | 94 | 41 pre-regulation USGS daily; 53 daily mass balance |
| 5-Day | 94 | 41 pre-regulation USGS daily; 53 daily mass balance |

Table 4-3. Assembled record, values and sources by duration.

## 4.7 Peak Duration Flow Consistency Review

A shorter-duration flow lower than a longer-duration flow for a single peaked event is physically impossible but can result from edge cases. Fifteen inconsistencies appeared, fourteen from a source mismatch (typically a regression-estimated peak below the same year's mass-balance 1-day value) and one from a double-peaked March event whose 5-day window spans both peaks while no 3-day window can (WY 2014). The record was made monotonic by anchoring each water year at its 5-day value and raising each shorter duration wherever a longer one exceeds it, cascading upward. Ten values across nine water years were raised , eight of them regression-estimated peaks lifted to their 1-day value, with the largest changes, 64, 19, and 19 percent, in WY2001, 1985, and 2006, all among the smallest years in the record that do not strongly influence the upper end of the final frequency curves.

## 4.8 Flow Frequency Analysis

The assembled record was analyzed in HEC-SSP with the Bulletin 17C expected moments algorithm, fitting a log-Pearson Type III distribution to each duration and weighting station skew against a published regional value of −0.07 for the peak (Mastin, 2016), and 0.09 / 0.07 / 0.00 for the 1-, 3-, and 5-day durations (Lind, 2020). The 5-day value is interpolated between that study's bracketing 3- and 7-day regional skews.

Water years 1969–1973 have observed regulated peaks but no unregulated basis, so they were entered as censored. The peak analysis reports 95 systematic events within a 100-year period, with five flagged missing. No low outliers were identified by the multiple Grubbs-Beck procedure.

Station skews are slightly negative for every duration (−0.158 to −0.054); weighting against the regional values gives adopted skews of −0.135, −0.052, −0.009, and −0.031 for the peak, 1-, 3-, and 5-day durations, with equivalent record lengths of 100–112 years after censoring. Table 4-4 gives the fitted parameters and event counts; Figure 4-2 shows the four adopted curves. Individual curves with confidence limits and plotted observations appear in Appendix C, and the full ordinates in Appendix B.

| Parameter | Peak | 1-Day | 3-Day | 5-Day |
| --- | --- | --- | --- | --- |
| Mean of logs | 4.778 | 4.743 | 4.660 | 4.593 |
| Standard deviation of logs | 0.202 | 0.203 | 0.192 | 0.179 |
| Station skew | -0.158 | -0.152 | -0.076 | -0.051 |
| Regional skew | -0.07 | 0.09 | 0.07 | 0.00 |
| Weighted skew | -0.135 | -0.052 | -0.007 | -0.029 |
| Adopted skew | -0.135 | -0.052 | -0.009 | -0.031 |
| Mean square error, at-site skew | 0.064 | 0.064 | 0.060 | 0.059 |
| Grubbs-Beck critical value | 0 | 0 | 0 | 0 |
| Equivalent record length (years) | 103.7 | 100.4 | 106.3 | 112.0 |
| Systematic events | 95 | 94 | 94 | 94 |
| Historical events | 0 | 0 | 0 | 0 |
| High outliers | 0 | 0 | 0 | 0 |
| Low outliers | 0 | 0 | 0 | 0 |
| Zero events | 0 | 0 | 0 | 0 |
| Missing events | 5 | 5 | 5 | 5 |
| Historical period (years) | 100 | 99 | 99 | 99 |

Table 4-4. Distribution parameters and event counts, adopted HEC-SSP analyses.

Figure 4-2. Adopted unregulated frequency curves at Castle Rock, water years 1927 through 2026, all four durations.

# 5. Regulated Flow Frequency at Castle Rock

The regulated peak flow frequency curve adds the effect of Mossyrock Dam on peak flows at Castle Rock. Mossyrock is not only operated for flood risk management—it is also operated for hydropower production. Mossyrock has frequently drafted deep into the power pool throughout the period-of-record resulting in flood space that is not guaranteed. While this additional space can be useful to reduce flows downstream, it is incidental rather than dependable. This additional space should not be relied on to provide a FRM benefit for planning purposes. EM 1110-2-1415 states that operations for project purposes other than FRM may provide incidental storage space, but the effect of this space on FRM should be estimated very conservatively. Therefore, the observed regulated flows were adjusted to remove these incidental effects and instead assume that the reservoir is operated for FRM only. This assumption is necessary to create a homogenous record. After adjusting the regulated peaks (5.1), they were paired with the unregulated peak of the same event (5.2) to build a relationship between unregulated peak flow and regulated peak flow. Synthetic floods were used to estimate extreme events not observed in the period of record (5.3) and add additional definition to the upper end of the relationship. The final regulated frequency curve is developed by applying the unregulated-regulated relationship to the peak unregulated frequency curve (5.4–5.5).

## 5.1 HEC-ResSim Flood Operation Calibration

Operations in the Corps Watershed System Model (CWMS) ResSim watershed were used as the basis for operations simulated in this study. Minimum, normal max evacuation, and special flood releases were re-assessed to calibrate flood operations for large events. Validating against observed operations was difficult because Mossyrock consistently releases well above its minimum releases for power generation and as a result pool elevations are frequently well below the standard minimum flood pool. However, for large events, starting a simulation from the observed elevation, adhering to the downstream control point regulation goals, and estimating special release parameters to prevent overtopping lead to reasonable validation. Figures 5-1, 5-1, and 5-3 show validations for the largest 3 events with sufficient hourly data to model the system spanning the POR 1990 to present. The largest and third largest events simulated, 1996 and 2015, match the observed peak and 5-day volumes reasonably well whereas the second largest validation event, 2009, shows an example of less-than-ideal validation. Section 5.2 describes a regulated peak flow adjustment capable of reducing the error associated with modeling discrepancies.

Figure 5-1. Flood simulation validation 1996.

Figure 5-2. Flood simulation validation 2009.

Figure 5-3. Flood simulation validation 2015.

## 5.2 Adjusted Regulated Peak Record

Mossyrock provides 357,000 acre-feet of authorized flood storage in Riffe Lake between the winter rule curve elevation of 749 feet and top of flood pool at 782 feet. The observed Mossyrock record shows the reservoir frequently drafted below the 749 feet, so a peak from a year that started low overstates the attenuation the project would provide from a flood risk management operation alone. Two period-of-record ResSim simulations, using the same observed hydrology and current release rules and differing only in starting pool, one from the Water Control Manual rule curve, one from the observed elevation at event onset, isolate that effect. Where the rule-curve start yields the higher regulated peak, the difference is added to the observed peak. The adjustment is one-sided, since a negative difference means the historical operation was already at least as conservative as the rule curve requires. Taking the difference between simulated releases reduces errors associated with an inability to model the exact operations in a particular year.

adjusted peak = observed peak + (simulated RC-start peak − simulated observed-start peak), where the RC-start peak is higher

Fifty-one water years (1974-2024) were assessed. Thirty-eight received an adjustment, with a median of 11,000 cfs and a range of 250-24,000 cfs. Ten were screened out. Seven were screened due to a timing mismatch between the observed and simulated peaks, two because the regulated peak exceeded the unregulated peak (physically possible only if release exceeds inflow during a large flood), and WY1980 which was the result of the Mount St. Helens lahar. 5 years received no adjustment, 4 where the two simulations produced the same peak and WY1976 where the observed pool started above the rule curve so the observed peak is not reduced.

## 5.2 Critical Duration

Developing an unregulated-regulated relationship requires choosing which unregulated duration best corresponds to the regulated peak (Table 5-1, Figure 5-4). Both the instantaneous peak and the 1-day duration are strong predictors. The peak unregulated flow is adopted for the transform, which is standard practice per EM 1110-2-1619.

| Duration | n | R² | Log R² | Log slope |
| --- | --- | --- | --- | --- |
| Peak (1-hr) | 44 | 0.870 | 0.847 | 0.602 |
| 1-Day | 44 | 0.853 | 0.830 | 0.605 |
| 3-Day | 44 | 0.792 | 0.783 | 0.633 |
| 5-Day | 44 | 0.733 | 0.734 | 0.663 |

Table 5-1. Adjusted regulated against unregulated flow by duration, water years 1974 through 2024.

Figure 5-4. Adjusted regulated against unregulated flow, by duration.

## 5.3 Synthetic Flood Ensembles

The period of record is not sufficient to estimate the unregulated-regulated relationship for extreme high flows. Synthetic floods were created by scaling the largest 12 events ranked by their unregulated flow estimate at Castle Rock and scaled to target the 100-, 250-, 500-year, and 20% above the 500-year unregulated peaks, resulting in 48 synthetic events of varying shapes and sizes.

Each source storm is scaled to hit its target unregulated peak and 5-day volume (within 5 percent), with Mossyrock inflow and Castle Rock local scaled by the same factor to preserve the observed coincidence. The events span regulated peaks of 81,000–267,000 cfs.

Figure 5-5. The twelve observed source storms, unscaled and aligned on each storm's own peak.

Figure 5-6. Scaling one source storm. Each member is scaled to its own target peak and 5-day volume; the shaded band is the window over which the 5-day volume is matched.

## 5.4 Unregulated-Regulated Transform

The regulated frequency curve is drawn through the observed and synthetic regulated flows with a locally weighted regression. The historical and synthetic pairs of regulated and unregulated flows provide the basis for drawing the line. The scatter is the result of storms with similar peak volumes but different shapes and starting pool elevations resulting in different regulated outcomes. The median estimated maximum regulation capacity of the reservoir is taken as ~325,000 cfs, which falls within the extrapolated uncertainty range.

Uncertainty is measured as the spread of the event pairs about the line fitted by a local area regression, in log space, assessed separately above and below the line so the asymmetry is preserved. Figure 5-7 shows the resulting band at the 5 to 95 percent level associated with the variation in regulation only. Section 5.8 combines the regulation uncertainty with the uncertainty introduced by mapping regulated flows to unregulated flow frequencies which carry their own uncertainty.

Figure 5-7. Regulated against unregulated peak at Castle Rock. The line is drawon from a locally weighted regression extended to meet an estimated regulated=unregulated convergence at ~ 325,000 cfs.

## 5.5 Regulated Frequency Curve

Each ordinate of the adopted unregulated curve is read through the transform, inheriting its AEP. The regulated flow at a given AEP is what the projects produce from the unregulated flow of that AEP (Table 5-2). The reduction peaks at about 34 percent near the 2 percent AEP. It is smaller at more frequent flows, where events are already largely contained, and falls away again at rarer flows, where storage is exhausted and the project must pass more of the inflow, reaching about 16 percent at the 0.1 percent AEP.

| Event | AEP | Unregulated (cfs) | Regulated (cfs) | Reduction | Regulated, 5%-95% (cfs) |
| --- | --- | --- | --- | --- | --- |
| 2-year | 0.500 | 60,529 | 50,811 | 16% | 39,578 - 60,614 |
| 10-year | 0.100 | 108,618 | 74,786 | 31% | 57,545 - 91,370 |
| 50-year | 0.020 | 153,556 | 100,670 | 34% | 73,092 - 143,827 |
| 100-year | 0.010 | 174,075 | 116,809 | 33% | 83,114 - 172,352 |
| 200-year | 0.005 | 195,977 | 140,273 | 28% | 98,137 - 211,031 |
| 500-year | 0.002 | 227,900 | 178,194 | 22% | 123,407 - 272,253 |
| 1000-year | 0.001 | 255,074 | 213,481 | 16% | 147,271 - 330,843 |

Table 5-2. Adopted regulated frequency curve at Castle Rock.

Figure 5-8. Adopted regulated and unregulated frequency curves at Castle Rock

## 5.6 Regulated Flow Frequency Uncertainty

The regulated flow at a given AEP carries uncertainty from two independent sources, the unregulated frequency estimate itself, and the uncertainty in the unregulated-regulated transform. Uncertainty from the unregulated frequency analysis is carried through to the regulated curve following EM 1110-2-1619 (USACE, 2025). The uncertainty band is treated as Normal on the upper and lower side of its best estimate separately, which is a reasonable approximation and preserves the asymmetry in both the HEC-SSP confidence limits and the unregulated-regulated uncertainty. HEC-SSP reports its own limits at 5 and 95 percent. Those are converted to a standard deviation, combined with the transform standard deviation by root-sum-of-squares, and the single z = 1.645 for a 90 percent two-sided interval is applied once to the combined value, so every limit reported here is a 5th or 95th percentile. Appendix E lists every ordinate of the adopted regulated curve together with the two terms that produced its uncertainty band.

Upper95 = RegBest+ Unreg95-UnregBest2+(Reg95-RegBest2

Lower5 = RegBest+ UnregBest-Unreg52+(RegBest-Reg52

Upper / Lower = Regulated uncertainty including unregulated uncertainty

Unreg = Unregulated curve from SSP B17C

Reg = Regulated curve from local area regression (LOESS)

Best = Curve expected value

95 =  Curve 95th percentile

5 = Curve 5th percentile

# 6. Comparison with the 2009 Restudy

## 6.1 Unregulated Curves

Figure 6-1 places the adopted unregulated curves against the 2009 Cowlitz Hydrology Restudy. The 2009 study adhered to Bulletin 17B and a period of record spanning most of 1927-2009 whereas the current study used Bulleting 17C, added 17 years of peaks to the record, and benefited from an updated regional skew study. The two studies agree within about 3 percent at the median and diverge at the tails. The 2009 adopted positive skews (0.140–0.193) that bend its curves upward in the tail, whereas the adopted skews here are near zero or negative.

Figure 6-1. Adopted 2026 unregulated curves (solid) against the 2009 Cowlitz Hydrology Restudy (dashed).

## 6.2 Regulated Curve

Figure 6-2 places the adopted regulated curve against the 2009 Restudy. The most notable divergence occurs beyond the 0.2 percent AEP when the 2009 regulated curve sharply approaches the unregulated curve. The 2009 study relied on the shape of the 1933 event as the sole starting point for synthetic storm development, which, as noted in section 5.5, has a shape that is likely to cause the reservoir to fill and spill. The current study scaled 12 events, including the 1933 event, to produce synthetics of varying shapes and found the 1933 event response to be atypical compared to other events with different shapes but similar peak and 5-day volumes that the reservoir was more successful in regulating. Uncertainty was not defined for the 2009 regulated curve since only one synthetic event was simulated per AEP at the upper tail and the curve was fit directly through those points and then hand drawn to converge with the unregulated curve.

Figure 6-2. Adopted 2026 regulated curve against the 2009 Cowlitz Hydrology Restudy.

# 7. Extension Downstream to the Coweeman Confluence

The regulated frequency curve developed above applies at the Castle Rock gage, which drains 2,238 square miles. Between that gage and the Coweeman River confluence the Cowlitz picks up 238 square miles of additional drainage from Arkansas Creek, Ostrander Creek, the Coweeman River, and the ungaged local area along both banks. This section develops regulated peak frequency curves at each of those points.

## 7.1 Method

The contribution of local flow from a drainage area below Castle Rock to the regulated peak flow on the Cowlitz is estimated in two steps. First, the unregulated flow is estimated using a drainage area ratio comparing the local drainage area to that of Castle Rock. Once the unregulated flow is estimated, it is then scaled to account for peak runoff timing effects resulting from the mainstem Cowlitz peak being delayed by reservoir regulation and larger basin attenuation.

## 7.2 Local Contribution by Drainage Area

Coweeman peak unregulated flows for large events are roughly proportional to the drainage area ratio, although there are few large events to draw data from and ratios are much more variable for smaller events. The PRISM annual total precipitation records found a consistent proportionality without accounting for event size. A drainage area ratio is adopted, with no unit-runoff adjustment.

Figure 7-1. Peak flow, precipitation, and drainage area ratios between the Coweeman River and Cowlitz at Castle Rock

| Location | Drainage area (sq mi) | Incremental (sq mi) | Local as % of gage |
| --- | --- | --- | --- |
| Castle Rock gage | 2,238 | — | — |
| Below Arkansas Creek | 2,278 | 40 | 1.8% |
| Below Ostrander Creek | 2,335 | 97 | 4.3% |
| Below Coweeman River | 2,476 | 238 | 10.6% |

Table 7-1. Drainage area at each location.

## 7.3 Timing Adjustment

The tributaries do not crest when the regulated Cowlitz does. They peak first and are already receding when the regulated mainstem arrives, so only part of each tributary peak is present at the moment that governs the combined flow. Measuring the Coweeman flow at the hour of the regulated Castle Rock crest, as a fraction of its own peak in the same storm over 78 events (multiple events per year were assessed) resulted in an average scaling factor of 0.79. Larger events suggested a larger lag time resulting in a smaller local flow contribution, but the sample size was small, so a scaling factor of 0.8 was selected, matching what was used in the 2009 study.

| Castle Rock unregulated peak | Events | Median ratio |
| --- | --- | --- |
| 20,000–40,000 cfs | 52 | 0.81 |
| 40,000–60,000 cfs | 19 | 0.78 |
| Above 60,000 cfs | 7 | 0.41 |
| All events | 78 | 0.79 |

Table 7-2. Coweeman flow at the regulated Castle Rock crest, as a fraction of its own event peak.

## 7.4 Results

Regulated peak flows in table 8-3 are calculated as:

Qsite=QregCAS+QunregCAS*DAsite-DACASDACAS*L

Q =  Flow

reg = Regulated flow

unreg = Unregulated flow

DA = Drainage Area

L = Timing reduction factor

The drainage area ratio is applied to the unregulated flow at Castle Rock, because the tributaries below the gage respond to the storm rather than to the reservoir release. The resulting local contribution is added to the regulated flow at Castle Rock.

Complete ordinates with confidence limits are in Appendix F.

| AEP | Castle Rock gage | Below Arkansas | Below Ostrander | Below Coweeman |
| --- | --- | --- | --- | --- |
| 10% | 74,786 | 76,339 | 78,552 | 84,027 |
| 2% | 100,670 | 102,866 | 105,995 | 113,734 |
| 1% | 116,809 | 119,298 | 122,845 | 131,619 |
| 0.5% | 140,273 | 143,075 | 147,068 | 156,946 |
| 0.2% | 178,194 | 181,453 | 186,096 | 197,583 |
| 0.1% | 213,481 | 217,128 | 222,325 | 235,182 |

Table 7-3. Regulated peak flow at each location, cfs.

Figure 7-2. Regulated peak flow frequency at all four locations. The local contribution at each site is the incremental drainage area scaled off the unregulated curve and reduced by the 0.80 timing factor.

# Appendix A. Assembled Unregulated Record by Water Year

Flows are in cubic feet per second. Source abbreviations: USGS peak, the observed instantaneous annual peak; USGS daily, rolling maxima of the observed daily record; Holdout, the hourly unregulated record from the Mossyrock holdout; Regression, the two-day storage-change relation; Mass balance, the daily unregulated series.

| WY | Peak | Peak source | 1-Day | 1-Day source | 3-Day | 5-Day | Duration source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1927 | 41,100 | USGS peak | — | — | — | — | — |
| 1928 | 74,000 | USGS peak | 71,300 | USGS daily | 56,933 | 47,940 | USGS daily |
| 1929 | 21,800 | USGS peak | 21,100 | USGS daily | 19,633 | 18,980 | USGS daily |
| 1930 | 30,200 | USGS peak | 29,500 | USGS daily | 26,700 | 23,840 | USGS daily |
| 1931 | 65,900 | USGS peak | 63,200 | USGS daily | 51,200 | 40,760 | USGS daily |
| 1932 | 50,800 | USGS peak | 49,200 | USGS daily | 44,467 | 39,140 | USGS daily |
| 1933 | 53,000 | USGS peak | 52,200 | USGS daily | 44,467 | 43,900 | USGS daily |
| 1934 | 139,000 | USGS peak | 134,000 | USGS daily | 120,333 | 106,300 | USGS daily |
| 1935 | 61,800 | USGS peak | 58,100 | USGS daily | 53,800 | 45,540 | USGS daily |
| 1936 | 48,200 | USGS peak | 39,800 | USGS daily | 35,833 | 31,400 | USGS daily |
| 1937 | 49,000 | USGS peak | 47,200 | USGS daily | 40,167 | 34,640 | USGS daily |
| 1938 | 62,900 | USGS peak | 60,500 | USGS daily | 53,567 | 47,440 | USGS daily |
| 1939 | 43,600 | USGS peak | 39,800 | USGS daily | 29,867 | 27,840 | USGS daily |
| 1940 | 41,900 | USGS peak | 40,500 | USGS daily | 36,767 | 31,660 | USGS daily |
| 1941 | 26,400 | USGS peak | 22,100 | USGS daily | 19,633 | 16,038 | USGS daily |
| 1942 | 58,100 | USGS peak | 55,400 | USGS daily | 47,067 | 39,400 | USGS daily |
| 1943 | 60,500 | USGS peak | 58,300 | USGS daily | 46,867 | 39,020 | USGS daily |
| 1944 | 37,100 | USGS peak | 33,700 | USGS daily | 26,467 | 20,760 | USGS daily |
| 1945 | 45,300 | USGS peak | 44,200 | USGS daily | 36,333 | 30,340 | USGS daily |
| 1946 | 54,700 | USGS peak | 51,700 | USGS daily | 45,433 | 38,940 | USGS daily |
| 1947 | 85,100 | USGS peak | 82,400 | USGS daily | 80,767 | 74,880 | USGS daily |
| 1948 | 38,500 | USGS peak | 37,000 | USGS daily | 32,700 | 28,560 | USGS daily |
| 1949 | 51,500 | USGS peak | 43,300 | USGS daily | 30,167 | 28,580 | USGS daily |
| 1950 | 54,800 | USGS peak | 54,000 | USGS daily | 49,933 | 43,920 | USGS daily |
| 1951 | 67,200 | USGS peak | 63,900 | USGS daily | 61,467 | 54,440 | USGS daily |
| 1952 | 41,400 | USGS peak | 35,600 | USGS daily | 31,600 | 28,680 | USGS daily |
| 1953 | 60,000 | USGS peak | 50,000 | USGS daily | 44,000 | 40,400 | USGS daily |
| 1954 | 73,000 | USGS peak | 63,600 | USGS daily | 51,067 | 45,440 | USGS daily |
| 1955 | 50,100 | USGS peak | 43,300 | USGS daily | 33,800 | 29,020 | USGS daily |
| 1956 | 67,400 | USGS peak | 63,100 | USGS daily | 49,733 | 42,260 | USGS daily |
| 1957 | 43,200 | USGS peak | 40,400 | USGS daily | 37,400 | 34,980 | USGS daily |
| 1958 | 35,200 | USGS peak | 34,500 | USGS daily | 30,867 | 27,280 | USGS daily |
| 1959 | 57,800 | USGS peak | 52,900 | USGS daily | 44,733 | 39,140 | USGS daily |
| 1960 | 62,700 | USGS peak | 61,000 | USGS daily | 56,433 | 49,620 | USGS daily |
| 1961 | 64,500 | USGS peak | 59,000 | USGS daily | 46,133 | 43,520 | USGS daily |
| 1962 | 42,100 | USGS peak | 40,100 | USGS daily | 34,833 | 30,240 | USGS daily |
| 1963 | 66,900 | USGS peak | 59,500 | USGS daily | 52,900 | 41,640 | USGS daily |
| 1964 | 72,600 | USGS peak | 55,400 | USGS daily | 46,267 | 37,680 | USGS daily |
| 1965 | 83,900 | USGS peak | 79,100 | USGS daily | 68,933 | 60,120 | USGS daily |
| 1966 | 28,800 | USGS peak | 27,000 | USGS daily | 24,333 | 22,400 | USGS daily |
| 1967 | 65,000 | USGS peak | 58,800 | USGS daily | 48,167 | 38,520 | USGS daily |
| 1968 | 49,500 | USGS peak | 47,600 | USGS daily | 44,633 | 42,600 | USGS daily |
| 1974 | 112,486 | Regression | 112,486 | Mass balance | 92,392 | 74,846 | Mass balance |
| 1975 | 76,896 | Regression | 71,250 | Mass balance | 54,411 | 50,666 | Mass balance |
| 1976 | 124,946 | Regression | 110,190 | Mass balance | 106,676 | 91,422 | Mass balance |
| 1977 | 19,899 | Regression | 16,299 | Mass balance | 14,752 | 14,436 | Mass balance |
| 1978 | 131,885 | Regression | 110,965 | Mass balance | 94,793 | 74,569 | Mass balance |
| 1979 | 51,587 | Regression | 40,506 | Mass balance | 28,000 | 25,107 | Mass balance |
| 1980 | 101,895 | Regression | 42,737 | Mass balance | 37,116 | 32,514 | Mass balance |
| 1981 | 88,345 | Regression | 86,532 | Mass balance | 71,889 | 57,905 | Mass balance |
| 1982 | 84,883 | Regression | 77,900 | Mass balance | 62,871 | 60,342 | Mass balance |
| 1983 | 82,466 | Regression | 72,298 | Mass balance | 55,966 | 52,485 | Mass balance |
| 1984 | 57,352 | Regression | 57,352 | Mass balance | 46,702 | 38,367 | Mass balance |
| 1985 | 33,098 | Regression | 33,098 | Mass balance | 28,765 | 23,969 | Mass balance |
| 1986 | 95,135 | Regression | 83,652 | Mass balance | 62,163 | 52,287 | Mass balance |
| 1987 | 103,298 | Regression | 90,982 | Mass balance | 61,617 | 50,210 | Mass balance |
| 1988 | 61,876 | Regression | 59,525 | Mass balance | 39,029 | 30,728 | Mass balance |
| 1989 | 28,884 | Regression | 23,674 | Mass balance | 21,120 | 19,496 | Mass balance |
| 1990 | 124,824 | Regression | 95,400 | Mass balance | 76,113 | 59,391 | Mass balance |
| 1991 | 110,107 | Regression | 104,020 | Mass balance | 75,569 | 58,070 | Mass balance |
| 1992 | 51,845 | Holdout | 43,323 | Holdout | 38,240 | 35,850 | Mass balance |
| 1993 | 33,272 | Regression | 31,533 | Mass balance | 26,576 | 21,802 | Mass balance |
| 1994 | 26,300 | Regression | 26,300 | Mass balance | 24,710 | 22,556 | Mass balance |
| 1995 | 64,114 | Regression | 59,736 | Mass balance | 48,317 | 39,125 | Mass balance |
| 1996 | 176,855 | Regression | 170,280 | Mass balance | 139,807 | 108,126 | Mass balance |
| 1997 | 86,921 | Holdout | 83,113 | Holdout | 74,563 | 69,706 | Mass balance |
| 1998 | 38,925 | Regression | 30,823 | Mass balance | 23,581 | 22,965 | Mass balance |
| 1999 | 76,193 | Holdout | 68,206 | Holdout | 60,807 | 51,702 | Mass balance |
| 2000 | 73,070 | Regression | 72,890 | Mass balance | 58,190 | 43,549 | Mass balance |
| 2001 | 19,568 | Regression | 19,568 | Mass balance | 15,339 | 14,332 | Mass balance |
| 2002 | 66,027 | Regression | 64,512 | Mass balance | 49,168 | 40,567 | Mass balance |
| 2003 | 109,719 | Holdout | 101,254 | Holdout | 72,896 | 53,892 | Mass balance |
| 2004 | 56,223 | Holdout | 52,144 | Holdout | 43,452 | 34,949 | Mass balance |
| 2005 | 52,859 | Regression | 50,133 | Mass balance | 39,349 | 31,339 | Mass balance |
| 2006 | 74,082 | Regression | 74,082 | Mass balance | 62,808 | 53,945 | Mass balance |
| 2007 | 147,777 | Regression | 147,777 | Mass balance | 100,470 | 73,836 | Mass balance |
| 2008 | 72,455 | Holdout | 70,265 | Holdout | 54,504 | 41,446 | Mass balance |
| 2009 | 151,296 | Holdout | 139,073 | Holdout | 99,324 | 72,961 | Mass balance |
| 2010 | 27,802 | Holdout | 25,172 | Holdout | 24,888 | 23,204 | Mass balance |
| 2011 | 87,160 | Holdout | 85,731 | Holdout | 71,689 | 58,118 | Mass balance |
| 2012 | 57,286 | Holdout | 53,334 | Holdout | 40,865 | 34,419 | Mass balance |
| 2013 | 58,409 | Regression | 47,950 | Mass balance | 36,797 | 30,359 | Mass balance |
| 2014 | 60,268 | Holdout | 57,504 | Holdout | 46,396 | 46,396 | Mass balance |
| 2015 | 47,323 | Holdout | 43,517 | Holdout | 32,555 | 31,664 | Mass balance |
| 2016 | 135,790 | Holdout | 124,896 | Holdout | 92,957 | 72,910 | Mass balance |
| 2017 | 64,342 | Holdout | 62,516 | Holdout | 51,010 | 47,186 | Mass balance |
| 2018 | 54,188 | Holdout | 52,984 | Holdout | 47,285 | 39,300 | Mass balance |
| 2019 | 24,389 | Holdout | 24,389 | Holdout | 24,389 | 22,197 | Mass balance |
| 2020 | 77,812 | Holdout | 73,544 | Holdout | 60,465 | 51,297 | Mass balance |
| 2021 | 61,703 | Holdout | 56,924 | Holdout | 42,829 | 35,700 | Mass balance |
| 2022 | 77,833 | Holdout | 73,803 | Holdout | 60,851 | 50,050 | Mass balance |
| 2023 | 65,623 | Holdout | 61,507 | Holdout | 47,710 | 35,341 | Mass balance |
| 2024 | 81,192 | Holdout | 76,898 | Holdout | 59,644 | 48,589 | Mass balance |
| 2025 | 45,954 | Holdout | 41,975 | Holdout | 35,253 | 30,112 | Mass balance |
| 2026 | 101,833 | Holdout | 97,675 | Holdout | 94,669 | 77,439 | Mass balance |

Table A-1. Assembled unregulated record, water years 1927 through 2026.

# Appendix B. Unregulated Frequency Curve Ordinates

Computed curve, expected probability curve, variance of the log, and 5 and 95 percent confidence limits for every ordinate reported by HEC-SSP. Flows are in cubic feet per second.

| Duration | AEP | Variance of log | z | Computed | 5% limit | 95% limit | Expected probability |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Peak | 0.0001 | 0.0098 | 3.72 | 296,088 | 476,290 | 217,104 | 374,643 |
| Peak | 0.0002 | 0.0082 | 3.54 | 276,072 | 426,543 | 207,624 | 332,739 |
| Peak | 0.0005 | 0.0064 | 3.29 | 250,134 | 366,728 | 194,487 | 285,568 |
| Peak | 0.001 | 0.0052 | 3.09 | 230,884 | 325,578 | 184,044 | 255,074 |
| Peak | 0.002 | 0.0041 | 2.88 | 211,932 | 287,635 | 173,114 | 227,900 |
| Peak | 0.005 | 0.0029 | 2.58 | 187,282 | 241,929 | 157,818 | 195,977 |
| Peak | 0.01 | 0.0022 | 2.33 | 168,884 | 210,386 | 145,504 | 174,075 |
| Peak | 0.02 | 0.0016 | 2.05 | 150,625 | 181,177 | 132,434 | 153,556 |
| Peak | 0.05 | 0.0010 | 1.64 | 126,514 | 145,732 | 113,770 | 127,758 |
| Peak | 0.1 | 0.0007 | 1.28 | 108,041 | 121,015 | 98,383 | 108,618 |
| Peak | 0.2 | 0.0005 | 0.84 | 88,923 | 97,624 | 81,626 | 89,126 |
| Peak | 0.5 | 0.0005 | 0.00 | 60,585 | 65,857 | 55,691 | 60,529 |
| Peak | 0.8 | 0.0006 | -0.84 | 40,671 | 44,531 | 36,709 | 40,487 |
| Peak | 0.9 | 0.0009 | -1.28 | 32,826 | 36,401 | 28,836 | 32,573 |
| Peak | 0.95 | 0.0013 | -1.64 | 27,417 | 30,925 | 23,227 | 27,041 |
| Peak | 0.99 | 0.0030 | -2.33 | 19,408 | 23,026 | 14,842 | 18,564 |
| 1-Day | 0.0001 | 0.0109 | 3.72 | 299,447 | 479,405 | 221,040 | 386,105 |
| 1-Day | 0.0002 | 0.0092 | 3.54 | 276,822 | 426,187 | 209,249 | 338,896 |
| 1-Day | 0.0005 | 0.0072 | 3.29 | 247,988 | 362,832 | 193,367 | 286,800 |
| 1-Day | 0.001 | 0.0059 | 3.09 | 226,952 | 319,700 | 181,106 | 253,245 |
| 1-Day | 0.002 | 0.0047 | 2.88 | 206,550 | 280,306 | 168,605 | 223,780 |
| 1-Day | 0.005 | 0.0034 | 2.58 | 180,481 | 233,404 | 151,652 | 189,945 |
| 1-Day | 0.01 | 0.0026 | 2.33 | 161,378 | 201,443 | 138,445 | 167,060 |
| 1-Day | 0.02 | 0.0019 | 2.05 | 142,727 | 172,189 | 124,837 | 145,949 |
| 1-Day | 0.05 | 0.0012 | 1.64 | 118,581 | 137,189 | 106,070 | 119,937 |
| 1-Day | 0.1 | 0.0009 | 1.28 | 100,464 | 113,080 | 91,102 | 101,103 |
| 1-Day | 0.2 | 0.0006 | 0.84 | 82,073 | 90,457 | 75,165 | 82,324 |
| 1-Day | 0.5 | 0.0005 | 0.00 | 55,503 | 60,349 | 51,025 | 55,470 |
| 1-Day | 0.8 | 0.0006 | -0.84 | 37,319 | 40,830 | 33,739 | 37,176 |
| 1-Day | 0.9 | 0.0008 | -1.28 | 30,257 | 33,490 | 26,713 | 30,053 |
| 1-Day | 0.95 | 0.0011 | -1.64 | 25,414 | 28,563 | 21,767 | 25,130 |
| 1-Day | 0.99 | 0.0022 | -2.33 | 18,268 | 21,476 | 14,392 | 17,638 |
| 3-Day | 0.0001 | 0.0093 | 3.72 | 240,363 | 376,364 | 180,427 | 298,915 |
| 3-Day | 0.0002 | 0.0079 | 3.54 | 222,081 | 335,087 | 170,391 | 264,436 |
| 3-Day | 0.0005 | 0.0062 | 3.29 | 198,868 | 285,924 | 157,011 | 225,645 |
| 3-Day | 0.001 | 0.0051 | 3.09 | 181,994 | 252,436 | 146,786 | 200,289 |
| 3-Day | 0.002 | 0.0041 | 2.88 | 165,677 | 221,830 | 136,451 | 177,857 |
| 3-Day | 0.005 | 0.0030 | 2.58 | 144,898 | 185,363 | 122,577 | 151,689 |
| 3-Day | 0.01 | 0.0023 | 2.33 | 129,717 | 160,485 | 111,876 | 133,835 |
| 3-Day | 0.02 | 0.0017 | 2.05 | 114,932 | 137,687 | 100,941 | 117,352 |
| 3-Day | 0.05 | 0.0011 | 1.64 | 95,836 | 110,351 | 85,995 | 96,884 |
| 3-Day | 0.1 | 0.0008 | 1.28 | 81,533 | 91,456 | 74,157 | 82,033 |
| 3-Day | 0.2 | 0.0006 | 0.84 | 67,026 | 73,645 | 61,589 | 67,236 |
| 3-Day | 0.5 | 0.0004 | 0.00 | 46,042 | 49,861 | 42,513 | 46,036 |
| 3-Day | 0.8 | 0.0005 | -0.84 | 31,599 | 34,399 | 28,743 | 31,493 |
| 3-Day | 0.9 | 0.0007 | -1.28 | 25,945 | 28,543 | 23,109 | 25,784 |
| 3-Day | 0.95 | 0.0010 | -1.64 | 22,044 | 24,587 | 19,120 | 21,810 |
| 3-Day | 0.99 | 0.0020 | -2.33 | 16,230 | 18,847 | 13,097 | 15,706 |
| 5-Day | 0.0001 | 0.0085 | 3.72 | 181,018 | 276,627 | 138,058 | 225,377 |
| 5-Day | 0.0002 | 0.0072 | 3.54 | 168,477 | 248,520 | 131,223 | 200,703 |
| 5-Day | 0.0005 | 0.0057 | 3.29 | 152,387 | 214,676 | 122,003 | 172,807 |
| 5-Day | 0.001 | 0.0046 | 3.09 | 140,566 | 191,358 | 114,869 | 154,542 |
| 5-Day | 0.002 | 0.0037 | 2.88 | 129,023 | 169,831 | 107,577 | 138,364 |
| 5-Day | 0.005 | 0.0027 | 2.58 | 114,149 | 143,861 | 97,651 | 119,354 |
| 5-Day | 0.01 | 0.0020 | 2.33 | 103,144 | 125,913 | 89,882 | 106,301 |
| 5-Day | 0.02 | 0.0015 | 2.05 | 92,301 | 109,270 | 81,834 | 94,152 |
| 5-Day | 0.05 | 0.0010 | 1.64 | 78,089 | 89,024 | 70,643 | 78,888 |
| 5-Day | 0.1 | 0.0007 | 1.28 | 67,267 | 74,814 | 61,613 | 67,651 |
| 5-Day | 0.2 | 0.0005 | 0.84 | 56,108 | 61,221 | 51,864 | 56,269 |
| 5-Day | 0.5 | 0.0004 | 0.00 | 39,563 | 42,625 | 36,714 | 39,551 |
| 5-Day | 0.8 | 0.0005 | -0.84 | 27,812 | 30,120 | 25,439 | 27,720 |
| 5-Day | 0.9 | 0.0006 | -1.28 | 23,104 | 25,274 | 20,704 | 22,966 |
| 5-Day | 0.95 | 0.0008 | -1.64 | 19,810 | 21,961 | 17,294 | 19,612 |
| 5-Day | 0.99 | 0.0018 | -2.33 | 14,823 | 17,089 | 12,041 | 14,367 |

Table B-1. Frequency curve ordinates, all four durations.

# Appendix C. Unregulated Frequency Curves by Duration

Individual curves as produced by HEC-SSP: computed curve, expected probability curve, 5 and 95 percent confidence limits, and observed events at Hirsch-Stedinger plotting positions.

Figure C-1. Unregulated instantaneous peak frequency curve.

Figure C-2. Unregulated 1-day frequency curve.

Figure C-3. Unregulated 3-day frequency curve.

Figure C-4. Unregulated 5-day frequency curve.

# Appendix D. Adjusted Regulated Peak Record by Water Year

Observed, simulated and adjusted regulated peaks for every shared water year, with the screening result recorded for each. Flows are in cubic feet per second. WCM_RC and Obs_RC start the simulation at the water control manual rule curve and observed elevation, respectively.

| WY | Observed | WCM_RC | Obs_RC | Difference | Adjusted | Screen |
| --- | --- | --- | --- | --- | --- | --- |
| 1974 | 68,600 | 33,207 | 19,208 | 13,999 | 82,599 | reg_over_unreg |
| 1975 | 58,900 | 57,075 | 56,714 | 361 | 59,261 | ok |
| 1976 | 86,700 | 75,597 | 105,150 | -29,553 | 86,700 | ok |
| 1977 | 15,000 | 14,187 | 12,187 | 2,000 | 17,000 | ok |
| 1978 | 86,400 | 79,778 | 79,778 | 0 | 86,400 | ok |
| 1979 | 39,400 | 42,972 | 40,972 | 2,000 | 41,400 | ok |
| 1980 | 97,000 | — | — | — | 97,000 | Lahar |
| 1981 | 46,900 | 50,909 | 39,206 | 11,703 | 58,603 | ok |
| 1982 | 65,500 | 64,483 | 63,822 | 661 | 66,161 | ok |
| 1983 | 67,300 | 59,457 | 59,457 | 0 | 67,300 | ok |
| 1984 | 49,600 | — | — | — | 49,600 | different_event |
| 1985 | 23,600 | 24,934 | 13,724 | 11,210 | 23,600 | different_event |
| 1986 | 62,700 | 62,206 | 60,657 | 1,549 | 64,249 | ok |
| 1987 | 77,600 | 73,775 | 73,059 | 716 | 78,316 | ok |
| 1988 | 42,100 | 52,818 | 38,880 | 13,938 | 56,038 | ok |
| 1989 | 23,600 | 27,470 | 13,443 | 14,026 | 37,626 | ok |
| 1990 | 89,200 | 84,639 | 83,907 | 732 | 89,932 | ok |
| 1991 | 76,000 | 62,368 | 61,083 | 1,286 | 77,286 | ok |
| 1992 | 27,000 | 35,102 | 21,102 | 14,000 | 41,000 | ok |
| 1993 | 14,700 | 25,147 | 11,148 | 14,000 | 28,700 | ok |
| 1994 | 17,300 | — | — | — | 17,300 | different_event |
| 1995 | 47,500 | — | — | — | 47,500 | different_event |
| 1996 | 112,000 | 132,172 | 108,967 | 23,205 | 135,205 | ok |
| 1997 | 56,700 | 54,439 | 52,636 | 1,803 | 58,503 | ok |
| 1998 | 30,800 | 28,784 | 28,784 | 0 | 30,800 | ok |
| 1999 | 57,600 | 53,768 | 50,442 | 3,326 | 60,926 | ok |
| 2000 | 41,700 | 49,782 | 35,782 | 14,000 | 55,700 | ok |
| 2001 | 11,600 | — | — | — | 11,600 | different_event |
| 2002 | 51,400 | 53,728 | 45,573 | 8,155 | 59,555 | ok |
| 2003 | 69,000 | 62,340 | 62,077 | 263 | 69,263 | ok |
| 2004 | 38,100 | 48,301 | 34,302 | 14,000 | 52,100 | ok |
| 2005 | 29,600 | 40,940 | 26,912 | 14,029 | 43,629 | ok |
| 2006 | 56,200 | — | — | — | 56,200 | different_event |
| 2007 | 77,300 | 94,857 | 70,844 | 24,013 | 101,313 | ok |
| 2008 | 64,300 | 61,129 | 60,053 | 1,076 | 65,376 | ok |
| 2009 | 106,000 | 81,855 | 81,470 | 385 | 106,385 | ok |
| 2010 | 20,900 | 26,302 | 15,335 | 10,966 | 31,866 | ok |
| 2011 | 45,300 | 44,599 | 30,599 | 14,000 | 59,300 | ok |
| 2012 | 36,900 | 38,920 | 24,921 | 13,999 | 50,899 | ok |
| 2013 | 47,000 | 51,961 | 37,704 | 14,257 | 61,257 | reg_over_unreg |
| 2014 | 32,300 | 43,271 | 29,271 | 14,000 | 46,300 | ok |
| 2015 | 38,500 | 45,556 | 31,552 | 14,004 | 52,504 | ok |
| 2016 | 83,700 | 81,000 | 80,324 | 675 | 84,375 | ok |
| 2017 | 40,000 | 51,716 | 39,201 | 12,515 | 52,515 | ok |
| 2018 | 35,100 | 45,514 | 31,518 | 13,996 | 49,096 | ok |
| 2019 | 26,000 | — | — | — | 26,000 | different_event |
| 2020 | 47,400 | 51,580 | 51,580 | 0 | 47,400 | ok |
| 2021 | 45,600 | 50,963 | 42,650 | 8,313 | 53,913 | ok |
| 2022 | 63,900 | 62,387 | 61,747 | 639 | 64,539 | ok |
| 2023 | 34,400 | 44,819 | 30,826 | 13,993 | 48,393 | ok |
| 2024 | 44,800 | 52,638 | 42,979 | 9,659 | 54,459 | ok |

Table D-1. Adjusted regulated peak record, water years 1974 through 2024.

# Appendix E. Regulated Frequency Ordinates by Location

Regulated peak flow at each of the four locations of Section 7, with 5 percent and 95 percent confidence limits. Flows are in cubic feet per second. At the gage the adopted expected probability unregulated curve is shown alongside; downstream, the local contribution is shown, being the incremental drainage area scaled off the unregulated curve and reduced by the 0.80 timing factor. Confidence limits at the three downstream locations are from Castle Rock, translated by the local contribution — see Section 7.4.

| AEP | Unregulated | Regulated | 5% | 95% |
| --- | --- | --- | --- | --- |
| 0.9900 | 18,564 | 18,564 | 12,588 | 23,645 |
| 0.9500 | 27,041 | 27,041 | 19,919 | 33,288 |
| 0.9000 | 32,573 | 32,573 | 24,617 | 39,639 |
| 0.8000 | 40,487 | 40,487 | 31,147 | 48,765 |
| 0.5000 | 60,529 | 50,811 | 39,578 | 60,614 |
| 0.2000 | 89,126 | 64,243 | 49,641 | 75,204 |
| 0.1000 | 108,618 | 74,786 | 57,545 | 91,370 |
| 0.0500 | 127,758 | 85,561 | 64,196 | 112,707 |
| 0.0200 | 153,556 | 100,670 | 73,092 | 143,827 |
| 0.0100 | 174,075 | 116,809 | 83,114 | 172,352 |
| 0.0050 | 195,977 | 140,273 | 98,137 | 211,031 |
| 0.0020 | 227,900 | 178,194 | 123,407 | 272,253 |
| 0.0010 | 255,074 | 213,481 | 147,271 | 330,843 |
| 0.0005 | 285,568 | 256,186 | 175,670 | 403,253 |
| 0.0002 | 332,739 | 328,154 | 224,273 | 514,096 |
| 0.0001 | 374,643 | 374,643 | 251,502 | 602,655 |

Table E-1. Castle Rock gage, 2,229 square miles.

| AEP | Local | Regulated | 5% | 95% |
| --- | --- | --- | --- | --- |
| 0.9900 | 265 | 18,830 | 12,854 | 23,910 |
| 0.9500 | 387 | 27,428 | 20,306 | 33,675 |
| 0.9000 | 466 | 33,038 | 25,083 | 40,104 |
| 0.8000 | 579 | 41,066 | 31,725 | 49,344 |
| 0.5000 | 865 | 51,676 | 40,443 | 61,479 |
| 0.2000 | 1,274 | 65,517 | 50,915 | 76,479 |
| 0.1000 | 1,553 | 76,339 | 59,098 | 92,923 |
| 0.0500 | 1,827 | 87,388 | 66,023 | 114,534 |
| 0.0200 | 2,196 | 102,866 | 75,288 | 146,022 |
| 0.0100 | 2,489 | 119,298 | 85,603 | 174,841 |
| 0.0050 | 2,802 | 143,075 | 100,939 | 213,833 |
| 0.0020 | 3,259 | 181,453 | 126,666 | 275,511 |
| 0.0010 | 3,647 | 217,128 | 150,918 | 334,490 |
| 0.0005 | 4,083 | 260,269 | 179,753 | 407,336 |
| 0.0002 | 4,758 | 332,912 | 229,031 | 518,853 |
| 0.0001 | 5,357 | 380,000 | 256,859 | 608,012 |

Table E-2. Below Arkansas Creek, 2,278 square miles.

| AEP | Local | Regulated | 5% | 95% |
| --- | --- | --- | --- | --- |
| 0.9900 | 644 | 19,208 | 13,232 | 24,288 |
| 0.9500 | 938 | 27,979 | 20,857 | 34,226 |
| 0.9000 | 1,129 | 33,702 | 25,747 | 40,768 |
| 0.8000 | 1,404 | 41,891 | 32,550 | 50,169 |
| 0.5000 | 2,099 | 52,909 | 41,677 | 62,712 |
| 0.2000 | 3,090 | 67,333 | 52,731 | 78,295 |
| 0.1000 | 3,766 | 78,552 | 61,311 | 95,136 |
| 0.0500 | 4,430 | 89,991 | 68,626 | 117,137 |
| 0.0200 | 5,324 | 105,995 | 78,417 | 149,151 |
| 0.0100 | 6,036 | 122,845 | 89,150 | 178,387 |
| 0.0050 | 6,795 | 147,068 | 104,932 | 217,826 |
| 0.0020 | 7,902 | 186,096 | 131,309 | 280,155 |
| 0.0010 | 8,844 | 222,325 | 156,115 | 339,687 |
| 0.0005 | 9,902 | 266,087 | 185,572 | 413,155 |
| 0.0002 | 11,537 | 339,692 | 235,811 | 525,633 |
| 0.0001 | 12,990 | 387,633 | 264,493 | 615,645 |

Table E-3. Below Ostrander Creek, 2,335 square miles.

| AEP | Local | Regulated | 5% | 95% |
| --- | --- | --- | --- | --- |
| 0.9900 | 1,579 | 20,143 | 14,167 | 25,224 |
| 0.9500 | 2,301 | 29,341 | 22,220 | 35,589 |
| 0.9000 | 2,771 | 35,344 | 27,388 | 42,410 |
| 0.8000 | 3,445 | 43,932 | 34,591 | 52,209 |
| 0.5000 | 5,150 | 55,960 | 44,727 | 65,763 |
| 0.2000 | 7,582 | 71,825 | 57,223 | 82,787 |
| 0.1000 | 9,241 | 84,027 | 66,785 | 100,610 |
| 0.0500 | 10,869 | 96,430 | 75,065 | 123,576 |
| 0.0200 | 13,064 | 113,734 | 86,156 | 156,890 |
| 0.0100 | 14,810 | 131,619 | 97,923 | 187,161 |
| 0.0050 | 16,673 | 156,946 | 114,810 | 227,704 |
| 0.0020 | 19,389 | 197,583 | 142,796 | 291,642 |
| 0.0010 | 21,701 | 235,182 | 168,972 | 352,543 |
| 0.0005 | 24,295 | 280,481 | 199,965 | 427,548 |
| 0.0002 | 28,308 | 356,462 | 252,581 | 542,404 |
| 0.0001 | 31,873 | 406,516 | 283,376 | 634,528 |

Table E-4. Below Coweeman River, 2,476 square miles.

# References

England, J.F., Jr., Cohn, T.A., Faber, B.A., Stedinger, J.R., Thomas, W.O., Jr., Veilleux, A.G., Kiang, J.E., and Mason, R.R., Jr., 2019, Guidelines for determining flood flow frequency - Bulletin 17C (ver. 1.1): U.S. Geological Survey Techniques and Methods, book 4, chap. B5, 148 p.

Lind, G.D., Lamontagne, J.R., and Stonewall, A.J., 2020, Development of regional skew coefficients for selected flood durations in the Columbia River Basin, northwestern United States and British Columbia, Canada (ver. 1.1, October 2020): U.S. Geological Survey Scientific Investigations Report 2020-5073, 48 p.

Mastin, M.C., Konrad, C.P., Veilleux, A.G., and Tecca, A.E., 2016, Magnitude, frequency, and trends of floods at gaged and ungaged sites in Washington, based on data through water year 2014 (ver. 1.2, November 2017): U.S. Geological Survey Scientific Investigations Report 2016-5118, 70 p.

U.S. Army Corps of Engineers, 2025, Risk Assessment for Flood Risk Management Studies, EM 1110-2-1619, 29 September 2025.

U.S. Army Corps of Engineers, Portland District, 2009, Cowlitz River Hydrology Restudy.

U.S. Army Corps of Engineers, 2014, Water Control Manual, Mossyrock Project, Cowlitz River, Washington.

U.S. Army Corps of Engineers, Hydrologic Engineering Center, HEC-ResSim Reservoir System Simulation, and HEC-SSP Statistical Software Package.

U.S. Geological Survey, National Water Information System: streamflow and peak-flow records for station 14243000, Cowlitz River at Castle Rock, Washington.