Executive Summary for LOP report

Discharge-Frequency Analysis

Overview

Level of protection (LOP) for the Cowlitz River levees is related to the frequency of discharges experienced in the lower Cowlitz River.  This relationship is typically expressed as discharge-frequency curves at key points along the river.  Since the Cowlitz River discharges at Castle Rock and below are significantly impacted by flood control and power operations at Mayfield and Mossyrock Dams, regulated discharge-frequency curves are used to assess LOP.

The existing regulated discharge-frequency curve is from a hydrologic analysis presented as part of the 1997 Cowlitz River Flood Hazard Study.  In the draft 2008 LOP update, a hydrologic analysis was initiated by Portland District to create a new regulated discharge frequency curve using a more robust methodology and an updated period of record.  The approach taken in the 2008 analysis is finalized and presented in its entirety in the appendix of this report.

In the updated hydrology, the actual and estimated regulated discharges for the period of record at Castle Rock (1927 to 2009) are used to create the regulated discharge-frequency curve.  Measured regulated peak discharge data is used from the years of significant regulation (1969-2009).  For years prior to regulation, a combination of detailed routings for larger events and a regulated-unregulated relationship for events less than 90,000 cfs are used to estimate regulated peak data.

Synthetic hydrology was developed to supplement the 83-years of data and add resolution to the 0.01 to the 0.001 annual exceedence probability (AEP) range.  The synthetic analysis used unregulated gage data from throughout the basin to create a number of hypothetic storm patterns based on large historic storms, which were scaled to larger events than what has been seen in the basin during the recorded period.  A HEC-ResSim model of the Cowlitz basin was used to route the hypothetical events through the reservoirs to determine likely regulated peak discharges at Castle Rock.  The results were merged into a composite discharge-frequency relationship representing the mean frequency for a given discharge.

The new regulated discharge-frequency curve at Castle Rock is graphically drawn, first through the 83 plotted points as describe earlier in the report, and then using the results from the synthetic analysis beyond that.  The new regulated discharge-frequency curve at Castle Rock is shown in the following figure.

Discharge-frequency curves for flow-change locations below Castle Rock are calculated using the regulated frequency curve at Castle Rock and discharge-frequency data from regional regression analyses for the major local tributaries.  Tributary inputs are reduced by a constant percentage based on observed timing of peaks from historic events.  The final discharge-frequency flows from Castle Rock to the Columbia River are shown in the following table.

Uncertainty for the updated hydrology at Castle Rock is related to the length of the systematic record.  An equivalent years-of-record (EYR) of 90 years is calculated based on 83 years of data adjusted for the historical period and for additional uncertainty from estimating regulated peaks for pre-regulation years.  The same EYR is assumed for the three flow-change locations below Castle Rock.

Figure #. Updated regulated discharge-frequency curve for Cowlitz River at Castle Rock.

Table #. Regulated Peak Discharges for Cowlitz River at Castle Rock and at major tributary confluences below Castle Rock.

| Percent Chance Exceedance | Cowlitz River Peak Flow (cfs) |  |  |  |
| --- | --- | --- | --- | --- |
|  | at Castle Rock | below Arkansas Creek | below Ostrander Creek | below Coweeman River |
|  | RM 17.5 | RM 16.5 | RM 8.54 | RM 1.7 |
| 99 | 21,700 | 22,200 | 22,400 | 24,000 |
| 95 | 28,100 | 28,700 | 29,100 | 31,200 |
| 90 | 32,400 | 33,100 | 33,600 | 36,100 |
| 80 | 38,800 | 39,700 | 40,200 | 43,200 |
| 70 | 44,400 | 45,400 | 46,000 | 49,400 |
| 60 | 49,800 | 51,000 | 51,600 | 55,400 |
| 50 | 55,500 | 56,800 | 57,500 | 61,700 |
| 40 | 59,500 | 60,900 | 61,700 | 66,300 |
| 30 | 64,100 | 65,700 | 66,600 | 71,700 |
| 20 | 70,000 | 71,800 | 72,800 | 78,500 |
| 10 | 74,000 | 76,200 | 77,400 | 84,100 |
| 5 | 79,000 | 81,600 | 82,900 | 90,500 |
| 4 | 81,200 | 83,900 | 85,300 | 93,200 |
| 2 | 88,000 | 91,000 | 92,600 | 101,300 |
| 1 | 97,000 | 100,400 | 102,200 | 111,700 |
| 0.7 | 104,000 | 107,600 | 109,400 | 119,400 |
| 0.5 | 110,000 | 113,700 | 115,700 | 126,000 |
| 0.2 | 156,000 | 160,200 | 162,500 | 173,800 |
| 0.1 | 240,000 | 244,600 | 247,000 | 259,100 |
| 0.08 | 270,000 | 274,700 | 277,100 | 289,500 |
| 0.05 | 300,000 | 304,900 | 307,500 | 320,400 |
| 0.01 | 390,000 | 395,800 | 398,800 | 413,500 |

Appendix B

Discharge-Frequency Analysis

B.1. Background

Discharge-frequency curves are required to assess the level-of-protection (LOP) at the various levees along the bottom ~17 miles of the Cowlitz River from Castle Rock to Kelso/Longview, WA.  Since discharges at Castle Rock and below are significantly impacted by flood control and power operations at Mayfield and Mossyrock Dams, regulated discharge-frequency curves are used to assess LOP.

The existing regulated discharge-frequency curve at Castle Rock is from the 1997 Cowlitz River Flood Hazard Study.  The methodology used in the 1997 analysis consisted of creating an unregulated discharge-frequency curve using pre-regulation data (1927-1968) and modeled “natural” discharge data from period of significant regulation (1969-1996), and then using a relationship between regulated and unregulated peak discharges to calculate the regulated discharge-frequency curve.   The following figure shows the regulated-unregulated discharge relationship from the 1997 study and the points used to create the curve (1969 – 1996).  Regulated-unregulated pairs for year 1997 to 2009 are also shown in the figure.

Figure B1. Regulated-unregulated discharge relationship used in 1997 study.

A hydrologic analysis was initiated by Portland District (CENWP) in 2008 to update the discharge-frequency curve using a more robust methodology and an updated period-of-record (POR).  A draft of this analysis was presented in the 2008 LOP Update Summary.  The finalized hydrologic analysis and discharge-frequency updates are presented in this report.

In the current analysis, the regulated discharge-frequency curve at Castle Rock is graphically fit through a combination of measured and estimated regulated peak discharge data.  A synthetic hydrologic analysis is used to supplement the 83 years of measured and estimated data and add resolution to the discharge-frequency relationship for larger events outside of the range of what has occurred in recorded history.

B.2. System Description

The Cowlitz River Basin, shown in the figure below, drains the mountainous region between Mount St. Helens and Mt. Rainier to the Columbia River at Longview, WA.  The upstream-most levee is at Castle Rock where a USGS gage has been recording discharge data since 1927.  Approximately 47% of the area upstream of Castle Rock is above Mossyrock Dam where significant regulation has occurred since April 1968.  For reservoir simulation and routing purposes, the area is divided into five tributaries above Castle Rock including local flows.  The five locations coincide with the major, long-term USGS gages in the basin.

Figure B2. Cowlitz basin map including the dams, tributaries with drainage areas, ResSim river reaches, and key USGS gages.

An HEC-ResSim routing and reservoir simulation model of the Cowlitz Basin has been in use for several years by CENWP.  Its primary function has been to estimate unregulated peak discharges at Castle Rock.  In the current study, the HEC-ResSim model is used to simulate reservoir operations and route hypothetical tributary flows down to Castle Rock as part of the synthetic analysis.  The river and reservoir network is shown in the following figure.

Figure B3.  HEC-ResSim model of the Cowlitz Basin.

B.3. Unregulated and Regulated Discharge Data

B.3.1 Measured Data

The Cowlitz River at Castle Rock, WA gage (USGS 14243000) has been in operation since December 1926.  There is a systematic record length of 83 years, Water Years (WY) 1927-2009, available for the discharge-frequency analysis.  Maximum annual instantaneous peak discharge data and average daily discharge are available on the USGS website.  Hourly data are available for recent years on the USGS’s instantaneous data archive website.  Additional short-interval data have been provided by the USGS upon special request for the current and past studies.

The discharge data from the 83-year POR at Castle Rock are not homogeneous; stream flow has been significantly impacted since 1969 due to reservoir regulation in Riffe Lake (Mossyrock Dam) and Mayfield Lake.  The measured Castle Rock data can be divided into two homogeneous periods: natural (WYs 1927-1968) and regulated (WYs 1969-2009).

Substantial effort has been put into estimating unregulated data during the regulated period, and vice versa, in order to maximize POR for a particular dataset.  Additional long-term USGS gages in the Cowlitz basin are used in the analysis for simulating flood events and estimating regulated and unregulated discharges.  The following figure shows the gages used in the analysis.

Figure B4. USGS gages in the Cowlitz Basin used in the Castle Rock discharge-frequency analyses.

B.3.2 Estimated Unregulated Data

For both the 1997 analysis and the present analysis, “natural” or unregulated discharge data are necessary for creating unregulated discharge-frequency curves and for assessing the relationship between regulated and unregulated discharges.  In both analyses, reservoir storage data and discharge data from throughout the basin are used to simulate without-project conditions and calculate unimpaired hydrographs at Castle Rock for the largest storms of each year post-1968.  From these hydrographs, unregulated peak and volume data can be calculated.  Short interval data are needed to accurately calculate local flows between gaged locations and changes in storage in the reservoir.  In the present analysis, HEC-ResSim is used to route hydrographs and simulate no-regulation conditions.

Since the 1997 report, there have been two noteworthy events: the November 2006 and the January 2009 events.  The November 2006 flood event was a very large event concentrated in the Tilton and Upper Cowlitz basins.  The storm produced an unregulated peak discharge of 155,000 cfs at Castle Rock, which is the third largest in the POR.  At the beginning of the event, Riffe Lake (Mossyrock) was well below the rule curve at elevation 737 ft with 439,500 acre-feet of storage available for flood control.  The scheduled amount of storage would have been 200,000 acre-feet.  The extra flood storage capacity and the fact that the storm was concentrated above the reservoirs resulted in a drastic reduction in peak discharge at Castle Rock to just 77,300 cfs.

The flood in early January 2009 was very different than the November 2006 flood.  The unregulated peak discharge was estimated at 133,000 cfs, which is the 5th largest in the POR.  Prior to the event, the elevation of Riffe Lake was 712.5 feet on January 6, with available storage of 665,500 acre-ft, almost twice the maximum required storage of 360,000 acre-ft, and more than 200,000 acre-ft more than what was available in November 2006.  The regulated peak discharge measured at Castle Rock on January 8, 2009 was 106,000 cfs, which is the second highest regulated discharge recorded since regulation began in 1969.  The small degree of flood peak reduction (about 20%) in this case is attributed to the fact that the storm was largely concentrated below the reservoirs.

The following figure shows the maximum annual 1-day unregulated discharges for the POR at Castle Rock.  The unregulated discharges for years with significant regulation (1969 and later) in this figure are calculated using a basic routing model with daily data.

Figure B5. Maximum annual 1-day unregulated discharges for Cowlitz River at Castle Rock.

B.3.3 Estimated Regulated Data

Due to a lack of short interval data and intermittent gage records, performing a detailed simulation of all the pre-regulation years is not feasible.  The approach taken in this study is to simulate only the events/WYs that would impact the upper end of the regulated frequency curve where LOP is estimated, and to use a more generic, less accurate approach for the less impactful WYs.

To identify the potentially influential WYs, the annual maximum 1-day discharges from 1927 to 1968 are combined with the available calculated unregulated 1-day discharges from WYs 1969 to the present, and are ranked from largest to smallest in the following table.  The table includes the November 1995 event and the 2009 peak discharge calculated in the present study.

Table B-I. Regulated and Unregulated Discharges at Castle Rock from WYs 1927 to 2009.

Of this data set, only four storms from 1927 to 1968 produced unregulated discharges that would have likely enacted preventative flood regulation, i.e. those flows projected to be larger than 70,000 cfs at Castle Rock.  Of these events, only the December 1933 event (WY 1934) was above 90,000 cfs, the low end of the major flood damage level.  The next three largest events are the 1947, 1965, and 1928 events, ranking 13, 15, and 17, respectively.

Because this study is primarily concerned with the magnitude and frequency of large events that will affect estimates of the LOP, which are between the 0.01 and 0.005 AEP discharges, a detailed simulation was performed only for the December 1933 event.  For all other pre-regulated years, regulated peak discharges were estimated using a range of regulated-unregulated discharge relationships.

B.3.3.1 The December 1933 Event

The December 1933 flood event was comprised of two large flood waves spaced just 13- to 14-days peak-to-peak.  It produced by far the largest 15-day average discharge at Castle Rock in the POR, averaging just greater than the bankfull discharge, which defines the primary flow objective for Mossyrock Dam.  The first flood wave had a 1-day average discharge of 102,000 cfs and a 4-day average discharge of 85,500 cfs, the two of which would have ranked 7th and 3rd largest on record were they not followed by the larger flood wave.  The second flood wave had the second largest 1- through 4-day average discharges, and it had the largest 5- and 7-day runoff volumes on record, averaging 106,000 cfs and 96,000 cfs, respectively.  The peak discharge of 139,000 cfs is the largest flow measured at Castle Rock, and the 4th largest when including all of the estimated unregulated peaks that have occurred since regulation began in 1969.

The Mossyrock and Mayfield Dams Water Control Manual specify that the dams were designed to contain all floods of record including this, the largest recorded event at the time.  Following the water control manual, the regulated discharge at Castle Rock would be kept at or below bankfull flow by utilizing all of the flood storage capacity.  The design only looks at the second, larger wave and assumes full storage capacity at the beginning of the event.

A review of operating procedures during historical events revealed that Mossyrock Dam seldom aggressively evacuates storage immediately following an event; more commonly, water stored above rule curve is released at a steady rate between 15,000 and 25,000 cfs, sometimes at the powerhouse discharge of 10,000 cfs.  The largest spill in the history of Mossyrock Dam was 32,000 cfs during the November 1995 flood event.   It should also be noted that the USGS flood stage below Mayfield Dam occurs at 25,000 cfs, and that notifications to local authorities are required when releases will exceed 20,000 cfs.

The December 1933 event was modeled to determine what the peak regulated discharge could have been under more realistic operating conditions.  Hourly inflow hydrographs were created using daily and annual peak discharge data for Cowlitz River at Mossyrock and Castle Rock.  Various instantaneous peak discharge and time-of-peak data provided by the USGS were also used in shaping the hydrographs.  The hydrographs were created by passing through observed peaks and balancing average daily flows.  Hourly hydrographs for locals were calculated as the difference between the routed upstream hydrograph at Mossyrock and Castle Rock.  The following figure shows three hydrographs and the data used to create them.

Figure B6. Synthetic hourly hydrographs passing through instantaneous peaks and matching daily average discharges were used to simulate the December 1933 flood event.

An evacuation rate of 25,000 cfs was assumed for the period between the two flood waves.  This is a fairly aggressive rate but is assumed to be a conservative estimate of what the maximum discharge could have been.  Ideal and less-than-ideal release operation schemes were assumed for early release during the second, larger flood wave to assess the sensitivity of regulation on the downstream regulated peak.  From these different scenarios, a range of regulated peak discharges at Castle Rock were calculated.

Using an evacuation rate of 25,000 cfs brings the pool back down to an elevation of 759 feet, about 62% of full storage capacity available, by the time the second wave hits.  Without any preventative regulation, the reservoir could have filled prior to the inflow to Mossyrock peaking, resulting in a downstream peak of 130,000 cfs.  This estimate, however, is assumed to be overly conservative.  Assuming some preventative regulation measures, i.e. if the regulators had decent forecasts and initiated early releases, the regulated peak discharge at Castle Rock would likely have ranged from about 100,000 to 120,000 cfs.  A more precise estimate, if possible, is not necessary for this analysis.

Below are two figures showing ResSim results for a scenario with moderate to poor regulation, showing a late ramp-up to 50,000 cfs as the pool approaches full, and the resulting fill-and-spill occurring approximately 12 hours after the inflow peak.  This scenario resulted in a regulated peak at Castle Rock of 118,000 cfs.

Figure B7. Mossyrock pool elevation (top graph) and the inflow and outflow (bottom graph) for a scenario.  The oscillations in dam outflow are a minor glitch in the model that have little to no impact on the downstream flows as the drastic flow changes are attenuated into a smooth hydrograph in the reach below Mayfield Dam.

Figure B8. The regulated discharge at Castle Rock for this scenario, shown in Red, peaks at 118,000 cfs, and is caused by Mossyrock Dam releasing inflows as the flood storage capacity in Riffe Lake is diminished.  The unregulated and cumulative local flows are shown in yellow and blue, respectively.

B.3.3.2 All Other Pre-Regulation Water Years

The following figure shows the regulated peak discharges plotted against their corresponding unregulated 1-day discharges for all regulated WY’s with the exception of 1981.  The green lines represent pre-regulation annual maximum 1-day discharges.  For unregulated 1-day discharges less than 90,000 cfs, the relationships m = 1.03, m = 0.85, and m = 0.67 represent maximum, average, and minimum relationships, respectively.  These relationships are used for estimating a range of regulated discharges from the observed, unregulated discharges.

Figure B9. Regulated-unregulated relationships used to estimate regulated peak discharges for pre-regulation years.

The following figure shows the expanded regulated discharge data as described above, ranked and plotted on a log-normal scale.  The figure includes three different series from the three regulated-unregulated relationships.  Plotting positions were recalculated for each series.  The figure also shows the discharge-frequency relationship of only the measured data from 1969 to 2009.    The December 1933 event is plotted assuming a regulated peak of 110,000 cfs, but an additional series was created to depict the range of 100,000 to 120,000 cfs.

Figure B10. Regulated frequency curves using various regulated-unregulated discharge relationships to estimate flows less than 90,000 cfs.

While there is considerable uncertainty with this method of estimating regulated flow, it is assumed that the range of possible regulated frequency curves is within the maximum (effectively no regulation) and minimum (optimum regulation) curves, and that middle curve, calculated using the best-fit regulated-unregulated relationship, is an adequate approximation of the median regulated discharge-frequency relationship for the 83 year POR.  How these curves affect the final regulated frequency curve is discussed later in the report.

3.3.4 Historical Data

The 1997 Cowlitz River Flood Hazard Study discussed historical flood data from “an old Corps of Engineers frequency curve” and from the Centennial edition of the Longview Daily News in 1953.  Noteworthy events that occurred prior to the Castle Rock gage include floods in 1896, 1906, 1917, and 1923.  The flood of 1896 was described at the time as the greatest flood of recorded history.  The peak discharges of the 1906, 1917, and 1923 events were estimated at between 45% and 60% of the February 1996 flood peak, and all three were less than the 139,000 peak discharge of the December 1933 flood.  It was concluded in the 1997 report that it would have been very unlikely that there were other floods between 1896 and 1927 greater than the unregulated February 1996 event, and therefore a historical period of 1896 to 1996 was used in calculating the unregulated peak frequency curve.  As it was not relevant to the methods used at the time, the 1997 report did not discuss assumptions about volume-frequency or regulated discharges.

To assess historical period assumptions to apply for the volume frequency curves (discussed in the following section), it is helpful to look at the largest volume-frequency flows.  The largest 1- through 4-day average discharges occurred with the February 1996 event, and all of these are greater than the peak discharge of the 1906 flood, the largest known flow in the period between 1896 and 1927, estimated at 112,000 cfs.  The largest 5-, 7-, and 15-day discharges (106,000, 96,000 and 75,000 cfs, respectively) occurred with the December 1933 event.  Looking at the large, long-duration events that have occurred in the gage record, the 1947 event stands out as being an unusually long-duration event with a relatively small peak discharge.  With this event, the ratios of 5- and 7-day average discharges to peak discharge are 0.88 and 0.77, respectively.  If the 1906 event had a similar shape, the 5- and 7- day durations would have equaled 98,600 and 86,200, both of which are lower than the corresponding discharges that occurred with the December 1933 event.  The average discharge of 75,000 with the December 1933 flood is a significant high outlier, and 40% greater than the next highest 15-day average discharge occurring with the December 1978 flood event.  It also should be noted that both of these events (December 1933 and December 1978) were comprised of two, large, back-to-back flood waves, as opposed to a sustained moderately high discharge.

The peak unregulated discharge for the 1906 flood, the largest known flow in the period between 1896 and 1927, was estimate to be 112,000 cfs, which is approximately the same discharge as the regulated peak of the 1996 event.  Assuming at least some flood reduction would have occurred with this event, the regulated peak would be less than that of the February 1996 event, and is fair to assume the same historical period applies for the regulated peaks as with the unregulated.

B.4 Synthetic Hydrology Analysis

Synthetic hydrology is used to estimate the upper end of the regulated discharge-frequency curve beyond the systematic record.  Several spatial and temporal storm patterns based on historic events are used to create not one but a suite of hypothetical storms for each of the following design annual exceedence probabilities (AEP): 0.10, 0.02, 0.01, 0.005, and 0.001.  The process of creating synthetic floods involves 1) creating unregulated peak and volume-duration frequency (VDF) curves at Castle Rock and at the main tributaries, 2) identify patterns in storm intensity from historic events, 3) translate storm patterns to inflow hydrographs, and 4) simulate reservoir regulation using HEC-ResSim.  With a storm pattern analysis, it is recognized that a given n-event regulated peak discharge at Castle Rock is not attributed to one specific storm configuration but is a composite of many different possible storms.  The approach taken in this study is to run a multitude of storm configurations for a given n-event to provide a better resolution on hydrologic possibility and some understanding of variability and certainty.

B.4.1. Unregulated Discharge-Frequency Curves

Peak discharge and VDF curves are used to link measured discharge to AEP, which for the purposes of this study is considered to be a normalized measure of storm intensity, and then from design AEPs back to discharges.  For this analysis, unregulated peak and VDF curves are created for the following locations:

- Castle Rock

- Upper Cowlitz Basin

- Tilton Basin

- Toutle Basin

- Castle Rock Locals

- Reservoir Locals

The VDF curves at Castle Rock are used to define flood volumes for a given design AEP flood.  Any hypothetical storm event should produce the design volume at Castle Rock after all of the hydrographs from the tributaries are routed to the downstream control point.  The tributary VDF curves serve two purposes: they are used 1) to help identify spatial variability in storm intensity across the entire basin, and 2) to re-distributing flood volumes from Cowlitz River at Castle Rock throughout the basin.

Log Pearson type III distributions were fit to the peak, 1-, 2-, 3-, 4-, 5-, 7-, and 15-day annual maximum discharges per guidelines specified in Bulletin 17B, including high/low outlier tests, historic period adjustments, and conditional probability adjustments.  Low outlier thresholds were applied to create a better graphical fit in the upper end of the curves.  The final adopted statistics for a given location were chosen to create smooth transitions between the different VDF curves.  In no cases were regional skew values applied to calculate weighted skews for peak flood frequency curves; it was noted that the skew values for each basin were reasonable and consistent with trends in skew values with the other VDF curves, as well as with the regional skew values of 0.3 and 0.0 for the two polygons covering the Cowlitz basin.  The data, historic data assumptions, flood frequency statistics, and curves are attached at the end of the report.  Annual maxima attributed to predominantly snowmelt-driven floods were screened for use in this analysis, for the purpose of maintaining homogeneity.

B.4.1.1. Castle Rock

The peak and VDF curves for Cowlitz River at Castle Rock were created using a combination of measured data from the pre-regulation period (1927-1968) and modeled data for years where flows were significantly impacted by Mossyrock Dam (1969-2009).  Measured data included annual maximum instantaneous discharges and daily data from the USGS website.  Duration maxima were calculated manually using Excel.

For years following the completion of Mossyrock Dam, detailed simulations were performed to estimate without-project or unregulated discharges.  CENWP regularly computes unregulated discharges as a part of annual monitoring.  The HEC-ResSim model described earlier is used for these simulations.  Inflow hydrographs at Cowlitz River near Randle (Upper Cowlitz), Tilton River, and the Toutle River are created first using available short-duration data, and then daily data are used in the absence of hourly data.  Locals were calculated directly for most years.  Changes in reservoir storage are calculated using twice-daily data.

B.4.1.2. Upper Cowlitz Basin

The VDF curves for Upper Cowlitz are created primarily using annual peak and daily flow data from the Cowlitz River near Randle, WA gage (#14233400) from the USGS website.  Coinciding with the construction of the Cowlitz Falls dam, the Cowlitz River near Randle gage was abandoned in 1994; however, the Cowlitz River near Kosmos, WA (#14233500) was in place just downstream.  The drainage areas are within 1% of eachother, so no adjustments were made to the Kosmos gage data.  The Cowlitz River at Mossyrock, WA gage (#14235000) is used to add an additional nine years between 1927 and 1948 when the gage was installed the Cowlitz River near Randle site.  This was done using correlations of duration maxima for a period of overlapping data from 1948 to 1960.

B.4.1.3. Tilton Basin

The VDF curves for the Tilton Basin are created using peak and daily discharge data from the existing gage at the Tilton River above Bear Canyon near Cinebar, WA (#14236200) from WY 1957 to the present, and from the old Tilton River near Cinebar, WA (#14226500), in place from WY 1942 to 1957, available on the USGS website.  A drainage area ratio (141/156) was applied to the latter to create a continuous dataset data set of 68 years.

B.4.1.4. Toutle Basin

The data used for the Toutle Basin VDF curves includes the entire period-of-record of both the Toutle River at Tower Road, WA gage (#14242580) and the Toutle River near Silver Lake, WA gage (#14242500) minus the five years following the Mount St. Helens eruption (WYs 1981 – 1985) for a total of 81 to 79 years.  For the homogeneous dataset, the Tower Road gage data is adjusted to the Toutle River near Silver Lake gage using the drainage area ratio (474/496).

To determine if the sediment retention structure (SRS) on the North Fork Toutle has an impact on the peak at Tower Road, timing of available peak flows were compared.  The USGS gage #14240525, NF Toutle River below SRS near Kid Valley WA, was in service for WYs 1990-1998 and 2001-2002.  Additionally, hourly data for the Tower Road gage is available for most of this period.  Comparisons of peak flow at the Kid Valley gage and the Tower Road gage for the 5 years of matching data showed that in all but one case, the peak at the Tower Road location occurred on the day prior to the peak at Kid Valley.  It was concluded that since the North Fork Toutle is peaking after Tower Road, the North Fork (and therefore, the SRS) does not impact the peak at Tower Road, and a detailed study of the SRS was not warranted.  However, it should be noted that the actual time of the Kid Valley peak was not available.  In 1996, Kid Valley and Tower Road peaked on the same day.  The peak at Tower Road was midday.  Since the hour of the Kid Valley peak was not available, it was not possible to determine the timing relationship.  However, since the Tower Road peak occurred at mid-day, there is a 50% chance that the Kid Valley peak occurred after the Tower Road peak.

B.4.1.5. Castle Rock Locals

Castle Rock Locals VDF curves were created using calculated data from a simple water balance.  Daily data were calculated by subtracting the measured average daily discharges at Castle Rock from the two upstream gages at Cowlitz River below Mayfield and Toutle River near Silver Lake.  With travel times of 2 and 7 hours from Tower Road and Mayfield to Castle Rock, respectively, no routing or lag was applied to the daily data.  Recognizing that the daily data calculated using such a course water balance is inadequate for estimating maximum annual daily discharges, it is assumed that over-time, the duration maxima could be useful, perhaps at 3- or 4-day and larger.  The daily data were compared with detailed hourly estimates of locals performed in the 1997 study.  It was confirmed that the annual maxima from the daily series was not usable, and the 2-day maxima were indeed questionable; however, the 3-, 4-, and 5-day maxima were showed some decent correlation, averaging 9%, 5% and 3% less than the hourly estimates, respectively.  The daily data were adjusted by these factors and then used to estimate flood frequency statistics.

Log Pearson analyses were preformed to create VDF curves for the 3- through 15-day durations.  The statistics from the VDF curves were compared with the VDF curves statistics from the other tributaries.  The mean, standard deviation, and skew values for the peak, 1-, and 2-day curves were estimated assuming a smooth continuation of the statistics with increasing/decreasing duration, similar to those from other tributaries.

As a check, the hourly hydrographs for the Castle Rock Locals calculated in the 1997 study were reduced to duration maxima and translated to AEPs using the VDF curves described above.  These AEPs were checked against the AEPs of the other tributaries (discussed in more detail in the following storm patterns analysis section) to ensure that it was similar in magnitude, not consistently high or low.  The flow frequency statistics were then adjusted slightly to obtain more realistic AEP curves in the various historical events.

(Note - It is agreed that the daily data from which these annual maxima and VDF curves from are not ideal, but it is believed that these data provide a better estimate of the hydrologic characteristics of sub-basin than those created using the drainage area method applied to the Toutle River basin.  The drainage area method was not used in the present study due to several factors, including the large drainage-area ratio required to extend the record of gaged area to the total area data (838/474 = 1.77), the large difference in average annual precipitation (84 inches for the Toutle Basin and 53 inches for the Castle Rock Locals), and the large difference in basin characteristics, such as slope and location.  The drainage area method would produce the exact same VDF curves for the two sub-basins, only the mean would be shifted by a factor equivalent to the log of the drainage area ratio.  By using actual, albeit course data, differences are evident in the standard deviation and skews of the VDF curves, indicating substantially different basin characteristics beyond size, as would be expected.)

B.4.1.6. Reservoir Locals

The VDF curves for Reservoir Locals were estimated using the adopted statistics from the other VDF curves in this study.  Due to the size, location, topography, and average annual precipitation (AAP), the basin was assumed to behave somewhere between that of the Tilton basin and the Castle Rock; therefore, the standard deviation and skew values for each duration were calculated as the average of Tilton and Castle Rock Locals standard deviation and skews.  The mean values were calculated using empirical relationships relating mean to drainage area and AAP.  The following relationship worked well in producing strong correlations:

Mean logQ = log (DA*AAP^x)

Where mean log Q is the log mean of discharges, DA is the drainage area is square miles, AAP is the average annual precipitation in inches, and x is an AAP weighting factor, ranging from 0.35 for the peak curve and 1.0 for the 15-day curve.  These relationships were calibrated to create the best-fit polynomial regression through all sub-basins including Castle Rock Locals.

Figure B11. Relationships used to estimate the log mean discharge at Castle Rock Locals and Reservoir Locals from drainage area and average annual precipitation.

B.4.2. Storm Patterns Analysis

Seventeen historical rainflood events were analyzed for spatial and temporal variability in storm intensity.  Peak and duration maxima were collected for all 17 historic events and converted to AEPs using the peak and VDF curves described in the previous section.    Plots were created for each historic storm event showing the AEP for each duration at each tributary.  Also included in the plots are the frequency of the regulated peak discharge, approximated using the Weibul plotting position.  The following figure is an example of one of these historical event plots.  All 17 historic event plots are attached at the end of the report.

Note: For Castle Rock, maxima were calculated using hourly data wherever possible, instead of day-end averages, and AEPs were calculated using adjusted flood frequency statistics that account for the increase in discharges resulting from this calculation method.  This was done because day-end averages would not suffice when balancing the routed hydrographs discussed in the next section.

Figure B12. Example of historic event plots used to identify temporal and spatial variability in storm intensity.  This particular event is the WY 1997 event, a longer duration event concentrated in the higher elevation tributaries, more specifically in the Toutle Basin.

One thing apparent when looking at all of the historic event plots is the wide range of spatial and temporal patterns as well as how the AEPs of the tributaries and unregulated discharge at Castle Rock relate to the regulated peak AEP.  With the end-goal of creating a set of hypothetical storms for a design AEP with which we can estimate a range of peak regulated discharges at Castle Rock, it is necessary to create a process for defining a given event with a single representative AEP.  In the 2002 Sacramento and San Joaquin Comprehensive Study, the 3-, 5-, and 7-day AEPs were averaged to create a simplified representation for a given storm event, with each main stem location or tributary having exactly one AEP.  In the much smaller Cowlitz Basin, the average of the 1- through 3-day AEPs is a better approximation of the intensity of the event as relevant to regulated peak discharges.  This assumption applied to the 1997 event, shown in the previous figure, results in substantial difference with the average AEP of the 1-, 2-, and 3-day durations equal to 0.17 and the approximate regulated peak equal to 0.26.  This exercise applied with each of the historic events illustrates the inherent difficulty in creating and defining synthetic floods based on unregulated flood frequency and the flawed but necessary assumption that the regulated flood frequencies are directly related to unregulated flood frequency.

To begin to quantify temporal variability, the average of the 1-, 2-, and 3-day AEPs were calculated for each of the historic events for all locations.  The relative AEP for the peak and individual durations were calculated by dividing by the average AEP, and then all of the storm events were plotted on one graph for each tributary (excluding Reservoir Locals).  From this array of curves, three patterns were created to represent the possible temporal shapes, all of which have an equal possibility of occurring.  The three patterns represent short, medium, and long duration storms.  The following figure shows the patterns for the Cowlitz River at Castle Rock.

Figure B13. The three patterns used to describe different duration patterns at Castle Rock.  The red, orange, and yellow lines represent the short, medium and long duration patterns, respectively.  Individual historic flood events are in gray behind the five patterns.  Note, a synthetic analysis disregarding temporal variability would essentially assume a flat pattern at relative AEP equal to 1.

To identify patterns in spatial variability, the average AEP at each tributary is divided by the average AEP at Castle Rock.  This term “relative AEP” is useful in identifying the intensity in one tributary relative to another.  It also creates an easy metric for distributing a storm pattern based on design AEP at Castle Rock.  The following figure shows relative AEPs of the tributaries (excluding Reservoir Locals) for the 17 historic events.  Storms that are fairly uniform in intensity will be clustered near 1.0, and conversely, a storm that is largely concentrated in one or two tributaries will have a wide spread in relative AEPs.  The figure also shows the Castle Rock AEP and the regulated peak discharge frequency estimated using the Weibul plotting position.

One thing evident in this figure is, again, a high degree of variability in spatial patterns of storms.  The 1978, 1976 and November 1995 storm events could be described as being clearly concentrated in the Upper Cowlitz Basin, while the February 1996 and 1947 events clearly represent storms centered in the Toutle Basin.  Another type of pattern could be described as highland centered, with low relative AEPs in Upper Cowlitz and Toutle (2003 and 1997), versus lowland centered (2009, 1990, and 1972), which all produced remarkably large regulated peak discharges at Castle Rock when the average Castle Rock AEPs were fairly small.  There does not seem to be a strong correlation between Castle Rock AEP, duration, and centered location, with the exception that all of the lowland centered storms had short durations.

Figure B14. Spatial variability in storm intensity, shown here as the tributary AEP relative to the AEP at Castle Rock.  The events are sorted from largest to smallest, left to right.  Letters at the bottom indicate the duration pattern of the event (S, M, and L signify short, medium, and long duration temporal patterns), and the number indicates the associated spatial pattern, described in the following paragraph.

To describe the range of spatial patterns seen historically, 4 patterns were created that relate tributary AEP to the Castle Rock AEP, shown in the following figure.  A design AEP would be defined at Castle Rock and these patterns would then be used to define the average AEP at each tributary.  The AEPs for a given duration are then calculated using a corresponding duration pattern as described earlier.

Figure B15. The four spatial patterns used to describe possible spatial variations in storm intensity.

The final configuration of AEPs, or a “centering”, would typically be different than the initial configuration created from the design AEP, spatial pattern, and temporal pattern due to the need to create balanced hydrographs that produce the desired, design incremental volumes at Castle Rock after being routed.  The hydrographs and how they are balanced is described in the following section.  Below is an graphical example of a final, balanced centering.  All of the final centerings are attached at the end of the report.

Figure B16.  Graphical centering of a 0.01 AEP, Toutle-centered, short duration event.

B.4.3. Synthetic Hydrographs

Translating a centering to individual tributary hydrographs is done similarly to the methods applied in Hicke et al’s Sacramento San Joaquin Comprehensive Study, only scaled down for the smaller basin.  A single 7-day wave is used for the hypothetical floods in this study.  This duration is based on the shape and timing of large historic rain floods in the basin, which typically were seen at Castle Rock in the form of 5- to 7-day flood waves.

Flood volumes are translated to the synthetic hydrographs using incremental volumes from the VF curves.  The 1-, 2-, 3-, 4-, 5-, and 6+7-day discharges are converted to volumes by multiplying the average discharge by the duration, and then converting to incremental volumes by subtracting the 1-day from the 2-day, the 2-day from the 2-day, etc.  These incremental volumes are assigned to different 12-hour periods in a generic hydrograph.  The peak discharge was applied directly in the middle of the largest 1-day flow period.  The periods were smoothed to create a more natural hydrograph, but the incremental volumes were still adjusted to be equal to or greater than the design volumes by a maximum 4% (based on the difference between 24-hour versus day-end maxima, as discussed earlier in the Castle Rock unregulated frequency curve section.)

The timing of the peak for each tributary is based on one of three temporal patterns, each one representative of short, medium, and long duration events from the historic record.  The three patterns, shown in the following table, are defined timing of the peak relative to the peak on the Tilton Basin, which was consistently the first gage to peak.

Table B-II.  Relative time-of-peak in hours for the different duration patterns.

|  | Upper Cowlitz | Tilton | Toutle | Castle Rock Locals | Reservoir Locals |
| --- | --- | --- | --- | --- | --- |
| short | 8 | 0 | 1 | 1 | 0 |
| medium | 16 | 0 | 6 | 2 | 0 |
| long | 24 | 0 | 12 | 6 | 0 |

All tributary hydrographs are routed to Castle Rock using a coarse water balance approach in Excel, using basic smoothing and lag.  The maximum 24-hour discharge at Castle Rock is compared to the design AEP 24-hour discharge.  The spatial pattern is adjusted slightly until the routed volume is within 2% for the 1- through 5-day durations.  The duration patterns are then adjusted slightly to ensure a smooth hydrographs (i.e. the 5-day incremental volume cannot be greater than the 4-day incremental volume).  The spatial pattern is then adjusted again as necessary in an iterative process to achieve a smooth and balanced set of hydrographs.    The following figure is an example of a balanced synthetic hydrograph.

Figure B17. Tributary hydrographs and a coarsely-routed Castle Rock hydrograph of a medium duration, Toutle-centered, 0.01 AEP event.

B.4.5. Reservoir Routing

The calibrated HEC-ResSim model is used to route the synthetic hydrographs.  Routing coefficients are calibrated to match observed attenuation and lag using hourly data from multiple recent storm events.  Reservoir regulation is simulated for Mossyrock Dam only; the 21,000 acre-feet of storage capacity behind Mayfield Dam was assumed negligible compared to the 360,000 acre-feet behind Mossyrock.  The key factors affecting reservoir operations at Mossyrock Dam are the downstream flow at Castle Rock and the amount of storage available in Riffe Lake.  Per guidelines in the water control manual, the reservoir is to operate to keep Castle Rock at or below 70,000 cfs.  Additional criteria are added to simulate special curve operations when approaching the top of flood control space.

A starting elevation equivalent to the design winter flood control pool (745.5’) was chosen for each synthetic storm.  This is a conservative estimate considering that the lake is typically well below the rule curve during the extended rainflood season (November through February), averaging about 10 feet below the winter flood control pool (December through January).  It is noteworthy that the vast majority of annual maximum discharges occur between November and February, and all of the large rainfloods have occurred in this time period.  The following figure shows the lake elevation non-exceedence statistics for years WYs 1975-2009.

Figure B18. Riffe Lake elevation statistics show that the reservoir is typically below the rule curve for the entire flood season.

To reduce the amount of model runs, each set of tributary hydrographs for each spatial and temporal pattern for a given n-event were combined into one continuous hydrograph of multiple flood waves, each wave representing a different storm pattern.  Ample time was provided between flood waves to allow for the reservoir to evacuate any stored water.  Also included for each n-event flood wave series was a uniform flood event (no temporal or spatial variability).

B.4.6. Synthetic Hydrology Results

Both the regulated and unregulated hydrographs at Castle Rock calculated for each simulation (one per n-event) were output from the model.  The peak and duration maxima were calculated for each unregulated pattern/flood wave and converted to AEP.  These AEPs were always slightly different than the design AEP, due to the inability to balance the hydrographs exactly in Excel.  These re-computed AEPs were used in plotting the results.

The regulated hydrographs for each flood wave were interpreted individually, rather than simply choosing the maximum value for each flood wave.  Peak discharges were consistently related to either the locals downstream of the reservoir or a later peak caused by the reservoir reaching capacity, at which time the reservoir outflow equals the inflow.  Where the maximum discharge resulted from the locals, the peak value was assumed to equal the regulated discharge for that event.  When the highest peak occurred with the reservoir-filling peak, some discretion was used to estimate a realistic regulated peak discharge for the event.  In cases where a large decrease in discharge would separate a large local peak and an even larger reservoir-filled peak, more realistic regulated hydrographs were assumed to estimate a minimum and occasionally a maximum regulated peak discharge.  Following is an example of choosing both minimum and maximum regulated peak discharges from the model-output hydrographs.

Max = 210,000

Min = 175,000

Figure B19.  Regulated (blue) and unregulated (red) hydrographs for the 0.001 AEP, “Uplands” centered, short duration pattern and the range of realistic regulated peaks.    In this case, an average value of 193,000 would be the chosen regulated peak discharge for specific event.

For the 0.10 and 0.02 AEP events, the maximum discharge for a given flood wave was often a result of a third type of peak attributed to simulated, unrealistically aggressive drawdown following the local peak.  These peaks were discounted because it is assumed that reservoir storage would not be evacuated at a rate higher than the peak discharge resulting from the downstream locals.  In these cases, the maximum discharge would be taken from the local peak.

The final regulated peak discharges for all of the synthetic storms are shown in the following table.  The plotting position of the individual hypothetical storms, shown in italics in Table B-III, is equal to the minimum of the 1- through 3-day AEPs at Castle Rock, assuming that the defining intensity for a given hypothetical event coincides with the defining temporal pattern, i.e. the 1-day intensity is relevant to the short duration pattern, and the 3-day intensity is relevant to the long duration pattern.

Table B-III. Simulated regulated peak discharges (cfs) for all of the hypothetical storms.

For use in creating the final regulate discharge-frequency curve at Castle Rock, the individual storm events were consolidated into a composite curve representing the estimated median value, as shown in the following figure.  The composite results are calculated by weighting each storm pattern according to the number of occurrences seen from the seventeen historical events and taking the average of frequencies from each pattern for a given discharge.  The composite results for the design AEPs shown in the table below were interpolated on a log scale and rounded to the thousands.

Figure B20. Storm patterns at various design events and the calculated composite curve.

Table B-IV. Composite discharge-frequency results for the synthetic analysis.

| AEP | Discharge (cfs) |
| --- | --- |
| 0.020 | 90,000 |
| 0.010 | 105,000 |
| 0.005 | 124,000 |
| 0.002 | 160,000 |
| 0.001 | 190,000 |

B.5. Regulated Discharge-Frequency Curve at Castle Rock

The following figure shows the updated regulated discharge-frequency curve.  Also shown are the existing regulated discharge-frequency curve from the 1997 study and the unregulated peak discharge-frequency curve from the current analysis.  The updated regulated curve is graphically drawn, first through the 83 measured and estimated annual maximum regulated discharges, and then using the composite results from the synthetic analysis beyond that.  The measured and estimated points are plotted using Wiebul plotting position and a historical period of 114 years.  Tabulated results are shown in the following table.

Figure B21. Updated regulated peak discharge-frequency curve at Castle Rock.

Table B-5#.  Cowlitz River at Castle Rock peak regulated discharge-frequency curve.

| AEP | Discharge |
| --- | --- |
| (%) | (cfs) |
| 99 | 21,700 |
| 95 | 28,100 |
| 90 | 32,400 |
| 80 | 38,800 |
| 70 | 44,400 |
| 60 | 49,800 |
| 50 | 55,500 |
| 40 | 59,500 |
| 30 | 64,100 |
| 20 | 70,000 |
| 10 | 74,000 |
| 5 | 79,000 |
| 4 | 81,200 |
| 2 | 88,000 |
| 1 | 97,000 |
| 0.7 | 104,000 |
| 0.5 | 110,000 |
| 0.2 | 156,000 |
| 0.1 | 240,000 |
| 0.08 | 270,000 |
| 0.05 | 300,000 |
| 0.01 | 390,000 |

B.6. Regulated Discharge-Frequency Curves below Castle Rock

In the 1997 report, discharges downstream of Castle Rock were incrementally increased at Arkansas Creek (RM 16.50), Ostrander Creek (RM 8.54), and Coweeman River (RM 1.70) to account for additional local flow below Castle Rock.  The flow adjustments were calculated using drainage area ratios applied to the natural discharge at Castle Rock.  By using drainage area ratios, the locals below Castle Rock are assumed to have similar rainfall intensities and have identical hydrographs shapes as the entire Cowlitz basin above Castle Rock.  This results in an unrealistic overestimation of locals inputs below Castle Rock.    For the current update, flow frequency statistics are estimated for the three largest tributaries below Castle Rock (Arkansas Creek, Ostrander Creek, and Coweeman River) and discharges are adjusted for differences in hydrograph shape.

There are no long-term, systematic records of peak flow available for Arkansas Creek and Ostrander Creek.  Discharge frequency statistics are calculated for Arkansas Creek and Ostrander Creek using the USGS Streamstats (USGS StreamStats 2008) web-based tool.  The results of this analysis are shown in the following table.

Table B-VI.  Peak discharge frequency data for Arkansas and Ostrander Creeks.

| AEP | Arkansas Creek Drainage Area = 44.7 sq. mi. (cfs) | Ostrander Creek Drainage Area = 25.8 sq. mi. (cfs) |
| --- | --- | --- |
| 99.9 | 410 | 230 |
| 99 | 580 | 325 |
| 95 | 790 | 440 |
| 90 | 930 | 520 |
| 80 | 1,130 | 620 |
| 70 | 1,300 | 710 |
| 60 | 1,470 | 790 |
| 54 | 1,570 | 820 |
| 50 | 1,620 | 881 |
| 40 | 1,800 | 960 |
| 30 | 2,040 | 1,100 |
| 20 | 2,300 | 1,220 |
| 10 | 2,740 | 1,470 |
| 5 | 3,200 | 1,700 |
| 2 | 3,770 | 2,010 |
| 1 | 4,220 | 2,240 |
| 0.5 | 4,650 | 2,480 |
| 0.2 | 5,310 | 2,790 |
| 0.1 | 5,700 | 3,010 |

Source:  USGS StreamStats

Discharge-frequency data for Coweeman River were estimated using the 35 years of peak discharge data from 1950 through 1984, available for the USGS gage Coweman (Coweeman) River near Kelso (USGS station #14245000).  The peak discharge from the February 1996 flood is also available for this gage and it was used in the analysis as a historic peak flow.  The peak flow frequency data for Coweeman River is summarized in the following table.

Table B-VII.  Peak discharge-frequency data for Coweeman River at the gage and adjusted to the confluence with the Cowlitz River.

| AEP | Coweeman River Drainage Area = 119 sq. mi. (cfs) | Coweeman River at Cowlitz River Drainage Area = 127 sq. mi. (cfs) |
| --- | --- | --- |
| 99.9 | 1,360 | 1,420 |
| 99 | 1,930 | 2,000 |
| 95 | 2,600 | 2,690 |
| 90 | 3,020 | 3,140 |
| 80 | 3,620 | 3,760 |
| 70 | 4,110 | 4,270 |
| 60 | 4,580 | 4,750 |
| 54 | 4,850 | 5,040 |
| 50 | 5,040 | 5,240 |
| 40 | 5,550 | 5,760 |
| 30 | 6,140 | 6,380 |
| 20 | 6,900 | 7,160 |
| 10 | 8,070 | 8,380 |
| 5 | 9,150 | 9,500 |
| 2 | 10,500 | 10,900 |
| 1 | 11,500 | 11,900 |
| 0.5 | 12,400 | 12,900 |
| 0.2 | 13,700 | 14,200 |
| 0.1 | 14,600 | 15,200 |

Short interval gage data are not available at these sites to accurately assess the timing of the peaks on the tributaries relative to the peak flow on the Cowlitz River; however, based on their relative size, it is assumed they would have a shorter time-to-peak than the Cowlitz at Castle Rock.  To confirm this, discharge data for the nearby East Fork Lewis River near Heisson, WA gage (USGS 14222500) were used to estimate the timing of the Cowlitz tributary peaks relative to the Cowlitz River peak flow.  The East Fork Lewis River Basin is about 25 miles south-southeast of the Coweeman River Basin.  The drainage area at the East Fork Lewis gage is 125 square miles, similar to the Coweeman River Basin.  Data for 17 events from water years 1996 through 2007 were available for analysis.  Peak discharge on the East Fork Lewis occurred from 1 to 12 hours before the peak flow on the Cowlitz River at Castle Rock.  At the time of the Castle Rock peak flow, the flow on the East Fork Lewis ranged from 56% to 98% of its peak flow with an average of 80%.

Based on this information, 80% of the peak discharge at each tributary is added to the Cowlitz River peak discharge at Castle Rock to estimate the discharge-frequency data for the modeled flow-change locations on the Cowlitz River at Arkansas Creek, Ostrander Creek and Coweeman River.  For example, the 0.01 AEP Cowlitz River peak discharge at Ostrander Creek is the 0.01 AEP discharge at Castle Rock plus 80% of the 0.01 AEP discharge for Arkansas Creek plus 80% of the 0.01 AEP discharge at Ostrander Creek.  The final discharge-frequency data for the Cowlitz River below Castle Rock to the Columbia River confluence are summarized the following table.

Table B-VIII.  Cowlitz River peak regulated discharges below Castle Rock to the Columbia River confluence (cfs).

| AEP | below Arkansas Creek | below Ostrander Creek | below Coweeman River |
| --- | --- | --- | --- |
|  | RM 16.5 | RM 8.54 | RM 1.7 |
| 99 | 22,200 | 22,400 | 24,000 |
| 95 | 28,700 | 29,100 | 31,200 |
| 90 | 33,100 | 33,600 | 36,100 |
| 80 | 39,700 | 40,200 | 43,200 |
| 70 | 45,400 | 46,000 | 49,400 |
| 60 | 51,000 | 51,600 | 55,400 |
| 50 | 56,800 | 57,500 | 61,700 |
| 40 | 60,900 | 61,700 | 66,300 |
| 30 | 65,700 | 66,600 | 71,700 |
| 20 | 71,800 | 72,800 | 78,500 |
| 10 | 76,200 | 77,400 | 84,100 |
| 5 | 81,600 | 82,900 | 90,500 |
| 4 | 83,900 | 85,300 | 93,200 |
| 2 | 91,000 | 92,600 | 101,300 |
| 1 | 100,400 | 102,200 | 111,700 |
| 0.7 | 107,600 | 109,400 | 119,400 |
| 0.5 | 113,700 | 115,700 | 126,000 |
| 0.2 | 160,200 | 162,500 | 173,800 |
| 0.1 | 244,600 | 247,000 | 259,100 |
| 0.08 | 274,700 | 277,100 | 289,500 |
| 0.05 | 304,900 | 307,500 | 320,400 |
| 0.01 | 395,800 | 398,800 | 413,500 |

The method used for calculating locals below Castle Rocks assumes negligible peak attenuation and coincident frequency.  Also, the 80% reduction in peak essentially due to time-to-peak and basin size likely overestimates the discharges at the smaller tributaries of Arkansas and Ostrander Creeks, especially for extremely large events where the regulated peak discharge at Castle Rock would not occur with the locals but after the reservoir fills, resulting in a later surge well after the other larger tributaries have peaked.  The fact that the three tributaries account for only 83% of the area below Castle Rock counteracts the previous factors to some degree; however, the estimates are still considered to be slightly conservative.  Despite the relatively coarse methods used here, the updated estimates of local flow below Castle Rock are improvements upon the methods used in the 1997 study, which used the drainage area method applied to the Cowlitz River at Castle Rock to estimate local flows.  The current method considers basin-specific hydrology for the local tributaries and adjusts for coincident peak timing.  
B.7. Hydrologic Uncertainty

Hydrologic uncertainty for the regulated discharge-frequency curve at Castle Rock is estimated based the systematic record length adjusted for additional uncertainty for years where the regulated peak was estimated.  In this study, 42 years were measured data, 42 years were estimated using a regulated-unregulated relationship, and one year was estimated using a detailed reservoir routing.

Uncertainty due to these methods is assessed graphically by assuming minimum and maximum conditions.  For the WY 1934 event, the range of possible regulated discharges from the detailed routing analysis is depicted graphically at the plotting positions potentially affected, which in this case are the largest four events, plotted from 0.043 to 0.0087.  One standard deviation for the graphical frequency curve through this range is approximately 4,000 cfs,

The uncertainty due to using a regulated-unregulated relationship is addressed by plotting minimum and maximum frequency curves based on upper and lower bounds of regulated-relationship as shown in figure B10.  The difference between the regulated frequency curves is considerable at more frequent events; however, the curves are identical for events with AEPs less than 0.08.  Because the LOP is concerned primarily with discharges in the 0.01 to 0.005 range, it is assumed that the hydrologic uncertainty due to the application of the regulated-unregulated relationship is negligible.

In FDA, an equivalent years-of-record (EYR) is used to describe hydrologic uncertainty.  For the current update, the EYR used in FDA is back-calculated to include the uncertainty due to the use of the estimated regulated data.  For the purposes of the LOP update, the critical discharge frequency chosen for the uncertainty analysis EYR back-calculation is 0.01 AEP (1/100).  An EYR of 75 years effectively produces the uncertainty of an 83 EYR with an additional standard deviation of 4,000 cfs.

The additional 30 years of historical record beyond the systematic record increases certainty but at a significantly reduced value, estimated at 50% for this analysis.  An additional 15 years of EYR are added to the 75 years representing the observed period of record for a total of 90 years representing the EYR of this hydrologic study.  The 90 EYR at Castle Rock is applied for the entire reach including the three flow-change locations below Castle Rock.

B.8. Attachments

B.8.1. Unregulated Discharge-Frequency Curves

B.8.1.1. Cowlitz River at Castle Rock

B.8.1.2. Cowlitz River near Randle (Upper Cowlitz)

B.8.1.3. Tilton River

B.8.1.4. Toutle River

B.8.1.5. Castle Rock Locals

B.8.1.6. Reservoir Locals

B.8.2. Historical Storm Patterns

B.8.3. Synthetic Storm Centerings

B.8.3.1. 0.10 AEP Centerings

B.8.3.2. 0.20 AEP Centerings

B.8.3.3. 0.01 AEP Centerings

B.8.3.4. 0.005 AEP Centerings

B.8.3.5. 0.001 AEP Centerings