MEMORANDUM FOR RECORD

U.S. Army Corps of Engineers, Portland District

| Item | Description |
| --- | --- |
| Subject | Unregulated and Regulated Flow Frequency, Cowlitz River at Castle Rock, Washington (USGS 14243000) |
| From | Hydrology and Hydraulics Branch, Portland District |
| Date | August 2026 |
| Status | DRAFT — combines the separate unregulated and regulated memoranda into one document; Section 8 (Columbia River confluence extension) pending |

# 1. Purpose

This memorandum documents the development of a regulated peak flow frequency curve for the Cowlitz River at Castle Rock, Washington to inform levee fragility assessment and floodplain mapping in the lower Cowlitz valley. Unregulated curves are developed for the instantaneous peak, 1-, 3-, and 5-day durations (Section 4) and used to develop a regulated flow frequency curve reflecting current Water Control Manual flood risk management operations (Section 5).

# 2. Basin Description and Project Operation

The Cowlitz River drains the southern Washington Cascades and enters the Columbia River near Longview. The Castle Rock gage (USGS 14243000) sits in the lower valley, downstream of the Toutle River confluence, with a peak-flow record spanning water years 1927 through 2026. The total basin area above Castle Rock is 2,238 square miles, of which 1,170 square miles (roughly half) of headwaters are regulated by Mossyrock Dam. Mossyrock Dam. Completed in 1968, provides the only usable flood storage in the basin. Just downstream of Mossyrock is Mayfield Dam which acts as a re-regulating dam that smooths power peaking releases from Mossyrock providing no meaningful flood regulation of its own. Cowlitz Falls Dam, above Mossyrock, and the Sediment Retention Structure on the Toutle are both run-of-river.

Figure 2-1. Cowlitz River basin, showing the Castle Rock gage, the Mossyrock and Mayfield projects, and the Toutle and Tilton tributaries.

## 2.1 Previous Studies

The previously adopted unregulated and regulated frequency curves at Castle Rock both come from the 2009 Cowlitz Hydrology Restudy, which predates Bulletin 17C and, does not include the most recent 17 years of record, and used a limited set of synthetic hydrographs to inform the upper portion of the regulated flow frequency curve. Section 6 compares both curves against the 2009 study.

# 3. Data Used

Table 3-1 lists the datasets and models used and the purpose each served. The observed hydrologic records are shared by both analyses; the reservoir simulation model and its outputs are specific to the regulated study.

| Dataset or model | Period used | Purpose |
| --- | --- | --- |
| Mossyrock pool elevation, hourly | Nov 1974 – May 2026 | Reservoir storage change at hourly resolution; basis of the hourly unregulated peak estimates (5.2) |
| Mossyrock storage telemetry, hourly | Regulated era | Data quality screening |
| Mossyrock pool elevation, daily | Oct 1973 – present | Storage change for the regression predictors and the daily mass balance (5.3, 5.4); observed starting pool for the Obs_RC simulation (5.1) |
| Mossyrock elevation-storage rating table | 2014 Water Control Manual | Relates pool elevation to reservoir storage |
| Castle Rock streamflow, hourly (USGS 14243000) | Regulated era | Regulated flow at the study location; source of the regulated peak |
| Castle Rock streamflow, daily (USGS 14243000) | WY1927 – present | Pre-regulation durations; regulated daily flow used for the 3-, 5-, and select 1-day duration maxima |
| Castle Rock annual peak flows (USGS 14243000) | WY1927 – present | Pre-regulation peaks; regulated peak for years the hourly record does not cover; basis of the adjusted regulated peak record (5.1) |
| Mayfield outflow, hourly (USGS 14238000) | Regulated era | Upstream boundary for routing the Mossyrock regulation signal to Castle Rock |
| HEC-ResSim model, Mossyrock and Mayfield | Current (2014 WCM) rules | Routes inflow hydrographs through the projects for the adjustment (5.1) and the synthetic ensembles (5.3) |
| Reservoir inflow and Castle Rock local flow | WY 1929 – 2026 | Simulation inflows, back-calculated and volume-corrected |
| ResSim period-of-record simulation, rule-curve start (WCM_RC) | 98 water years | Regulated peak under current rules from a rule-curve starting pool |
| ResSim period-of-record simulation, observed start (Obs_RC) | 53 water years | Regulated peak under current rules from the observed starting pool |
| ResSim synthetic ensemble simulation | 48 members | Regulated response above the observed range (5.3) |

Table 3-1. Datasets and models used, and their purpose.

# 4. Unregulated Flow Frequency

Unregulated flow is the flow that would have occurred at Castle Rock without regulation from Mossyrock reservoir. For water years 1927–1968 the gage record reflects unregulated flows (Section 4.4). For the post 1968 regulated period the reconstruction uses an hourly holdout where the hourly elevation record is clean enough (Section 4.1) and otherwise uses a peak estimated by regression from the daily reservoir record (Section 4.2). The 3- and 5-day durations, needing only daily resolution, come from a daily mass balance in all years (Section 4.3). The basis for calculating unregulated flows from a regulated record relies on adding back the Mossyrock reservoir holdout ( reservoir storage change expressed as a flow rate) to the observed gage record downstream of the dam. A positive holdout indicates the reservoir filling.

## 4.1 Hourly Unregulated Record

The cleaned hourly pool elevation was converted to storage using the 2014 Water Control Manual rating curve and differenced to obtain an hourly holdout, smoothed with a 3-hour centered moving average and restricted to the October–March flood season. Days with fewer than four valid storage readings were omitted, since they cannot be relied on to capture the annual peak.

The reservoir signal was translated to Castle Rock using the SSARR routing parameters in Table 4-1, from the calibrated Cowlitz CWMS model. Mayfield outflow plus the holdout, and Mayfield outflow alone, were each routed to Castle Rock; the difference between the two routed hydrographs is the routed reservoir effect, added to observed Castle Rock flow to give the hourly unregulated estimate.

| Reach | KTS | n | Phases |
| --- | --- | --- | --- |
| Mayfield outflow to Cowlitz River above Toutle River | 5 | 0.1 | 5 |
| Cowlitz River above Toutle River to Cowlitz River below Toutle River | 1 | 0.2 | 1 |
| Cowlitz River below Toutle River to Castle Rock | 1 | 0.2 | 5 |

Table 4-1. SSARR routing parameters, from the calibrated lower Cowlitz routing model.

Twenty-three water years produced holdout-based peaks (1992, 1997, 1999, 2003, 2004, and 2008 through 2026) with earlier hourly pool elevation too unstable for use.

## 4.2 Peak Estimation by Storage-Change Regression

For years without a usable hourly record (roughly a third of the regulated era) the unregulated peak is instead estimated by regressing the regulated-minus-unregulated peak difference on the largest 1- to 4-day increase in Mossyrock storage near the event (within 7 days of the regulated peak), then subtracting the predicted difference from the observed regulated peak. This is preferred over regressing the unregulated peak directly. The predictor and the regulated peak are both available for every regulated year, and because the adjustment is a fraction of the flow being reconstructed, a given percentage error translates to a much smaller error on the resulting unregulated peak.

| Storage-change window | Slope | Intercept (cfs) | R² | Standard error (cfs) |
| --- | --- | --- | --- | --- |
| 1-day | -0.673 | -7,751 | 0.759 | 7,565 |
| 2-day (adopted) | -0.869 | -3,836 | 0.871 | 5,528 |
| 3-day | -0.983 | -4,537 | 0.853 | 5,896 |
| 4-day | -1.140 | -4,770 | 0.836 | 6,239 |

Table 4-2. Candidate regressions of the regulated-minus-unregulated peak difference on Mossyrock storage change, fit to 17 water years.

The two-day window was adopted (R² = 0.871, standard error ≈ 5,530 cfs, n = 17). Pairs whose regulated and unregulated peaks fell more than 72 hours apart were excluded as different storms. Thirty water years were filled by regression; for 1974–1991, where the hourly record does not reach, the regulated peak came from the USGS annual peak record and the peak date centered the storage-change window.

Figure 4-1. Adopted two-day storage-change regression. Points are labeled by water year.

## 4.3 Durations from the Daily Mass Balance

Daily unregulated flow is the observed daily flow plus the daily change in Mossyrock storage as a flow rate. No routing is applied since Mayfield-to-Castle Rock travel time is under a day. The 1-day duration flow is calculated from an hourly record, when available, to catch peaks that include midnight values. When no hourly data is available the 1-Day duration is calculated from daily values. The 3-Day and 5-Day durations were calculated from daily data.

## 4.4 Pre-Regulation Period

Before December 1968 the gage record is unregulated as observed. Peaks for water years 1927–1968 are the USGS observed instantaneous annual peaks (42 water years), and the 1-, 3-, and 5-day values are rolling maxima from the USGS daily record. Water year 1927 carries a peak only; the daily record begins later.

## 4.5 Record Extension and Historical Floods

Correlation-based extension from a nearby gage was considered but no suitable donor was found. Nearby long-record gages either do not predate Castle Rock by much, are themselves regulated over the period that would be used, or do not correlate strongly enough to justify the added estimation error. With roughly a century of systematic record already in hand, the potential gain was small. A search of USGS qualification codes, the 2009 Restudy, and published USGS/NWS accounts of major southwest Washington floods turned up no pre-1927 event at Castle Rock that could be quantified or reliably bounded, so no historical events or Bulletin 17C perception thresholds were used.

## 4.6 Assembled Record

For water years 1927–1968 the record is the observed gage record; for 1974–2026 the peak is the hourly holdout where available and the regression estimate otherwise, the 1-day value follows the same split, and the 3- and 5-day values always come from the daily mass balance (Table 4-3, Appendix A). Coverage runs from WY1927 through WY2026 except for WY1969–1973, which have no daily Mossyrock elevation record. Every included water year has a peak.

| Duration | Values | Sources |
| --- | --- | --- |
| Peak | 95 | 42 pre-regulation USGS peaks; 30 storage-change regression; 23 hourly holdout |
| 1-Day | 94 | 41 pre-regulation USGS daily; 30 daily mass balance; 23 hourly holdout |
| 3-Day | 94 | 41 pre-regulation USGS daily; 53 daily mass balance |
| 5-Day | 94 | 41 pre-regulation USGS daily; 53 daily mass balance |

Table 4-3. Assembled record, values and sources by duration.

## 4.7 Consistency Review

A shorter-duration flow lower than a longer-duration flow for a single peaked event is physically impossible but can result from methodological edge cases. Fifteen inconsistencies appeared, fourteen from a source mismatch (typically a regression-estimated peak below the same year's mass-balance 1-day value) and one, WY2014, from a double-peaked March event whose 5-day window spans both peaks while no 3-day window can. The record was made monotonic by anchoring each water year at its 5-day value and raising each shorter duration wherever a longer one exceeds it, cascading upward; pre-adjustment values are retained alongside the adopted ones. Ten values across nine water years were raised , eight of them regression-estimated peaks lifted to their 1-day value, with the largest changes, 64, 19, and 19 percent, in WY2001, 1985, and 2006, all among the smallest years in the record that do not strongly influence the upper end of the final frequency curves.

## 4.8 Flow Frequency Analysis

The assembled record was analyzed in HEC-SSP with the Bulletin 17C expected moments algorithm, fitting a log-Pearson Type III distribution to each duration and weighting station skew against a published regional value of −0.07 for the peak (Mastin, 2016), and 0.09 / 0.07 / 0.00 for the 1-, 3-, and 5-day durations (Lind, 2020). The 5-day value is interpolated between that study's bracketing 3- and 7-day regional skews.

Water years 1969–1973 have observed regulated peaks but no unregulated basis, so rather than omit them (shortening the record) or assign point estimates (asserting unsupported values), they were entered as censored: the peak analysis reports 95 systematic events within a 100-year period, with five flagged missing. No low outliers were identified by the multiple Grubbs-Beck procedure. The unregulated series is constructed rather than gaged, so its small years carry the same provenance as its large ones and there is no physical basis for treating the lower tail as unrepresentative.

Station skews are slightly negative for every duration (−0.158 to −0.054); weighting against the regional values gives adopted skews of −0.135, −0.052, −0.009, and −0.031 for the peak, 1-, 3-, and 5-day durations, with equivalent record lengths of 100–112 years after censoring. Table 4-4 gives the fitted parameters and event counts; Figure 4-2 shows the four adopted curves. Individual curves with confidence limits and plotted observations appear in Appendix C, and the full ordinates in Appendix B.

| Parameter | Peak | 1-Day | 3-Day | 5-Day |
| --- | --- | --- | --- | --- |
| Mean of logs | 4.778 | 4.743 | 4.660 | 4.593 |
| Standard deviation of logs | 0.202 | 0.203 | 0.192 | 0.179 |
| Station skew | -0.158 | -0.152 | -0.076 | -0.051 |
| Regional skew | -0.07 | 0.09 | 0.07 | 0.00 |
| Weighted skew | -0.135 | -0.052 | -0.007 | -0.029 |
| Adopted skew | -0.135 | -0.052 | -0.009 | -0.031 |
| Mean square error, at-site skew | 0.064 | 0.064 | 0.060 | 0.059 |
| Grubbs-Beck critical value | 0 | 0 | 0 | 0 |
| Equivalent record length (years) | 103.7 | 100.4 | 106.3 | 112.0 |
| Systematic events | 95 | 94 | 94 | 94 |
| Historical events | 0 | 0 | 0 | 0 |
| High outliers | 0 | 0 | 0 | 0 |
| Low outliers | 0 | 0 | 0 | 0 |
| Zero events | 0 | 0 | 0 | 0 |
| Missing events | 5 | 5 | 5 | 5 |
| Historical period (years) | 100 | 99 | 99 | 99 |

Table 4-4. Distribution parameters and event counts, adopted HEC-SSP analyses.

Figure 4-2. Adopted unregulated frequency curves at Castle Rock, water years 1927 through 2026, all four durations.

# 5. Regulated Flow Frequency

The regulated peak flow frequency curve is developed from observed regulated flows corrected to assume the reservoir operated to maintain the minimum conservation elevation. This assumption is necessary to create a homogenous record and is conservative because Mossyrock has frequently drafted deep into the power pool throughout the period-of-record resulting in flood storage that is not guaranteed. The adjusted regulated peaks (5.1) are paired with the unregulated peak of the same event (5.2). Synthetic floods are used to estimate extreme events not observed in the period of record (5.3). The final regulated frequency curve is developed by mapping regulated flows to the peak unregulated frequency curve (5.4–5.5).

## 5.1 Adjusted Regulated Peak Record

The observed Mossyrock record shows the reservoir frequently drafted below the flood-pool minimum, so a peak from a year that started low understates the attenuation the project would provide from its full flood-control authority. Two period-of-record ResSim simulations, using the same observed hydrology and current release rules and differing only in starting pool, one from the WCM rule curve, one from the observed elevation at event onset, isolate that effect. Where the rule-curve start yields the higher regulated peak, the difference is added to the observed peak. The adjustment is one-sided, since a negative difference means the historical operation was already at least as conservative as the rule curve requires.

adjusted peak = observed peak + (simulated RC-start peak − simulated observed-start peak), where the RC-start peak is higher

Of the fifty-one water years (1974–2024) were assessed, 38 received an adjustment (median 11,334 cfs, range 263–24,013). Screening removed 10, 7 for a timing mismatch between the observed and simulated peaks, 2 where the regulated peak exceeded the unregulated peak (physically only possible if release exceeds inflow during a large flood), and 1 for both. WY1980 is excluded outright because its peak is the result of the Mount St. Helens lahar.

## 5.2 Critical Duration

Mapping a regulated flow onto the unregulated curve requires choosing which unregulated duration corresponds to the regulated peak (Table 5-1, Figure 5-1). Both the instantaneous peak and the 1-day duration are strong predictors. The peak unregulated flow is adopted for the transform.

| Duration | n | R² | Log R² | Log slope |
| --- | --- | --- | --- | --- |
| Peak (1-hr) | 44 | 0.870 | 0.847 | 0.602 |
| 1-Day | 44 | 0.853 | 0.830 | 0.605 |
| 3-Day | 44 | 0.792 | 0.783 | 0.633 |
| 5-Day | 44 | 0.733 | 0.734 | 0.663 |

Table 5-1. Adjusted regulated against unregulated flow by duration, water years 1974 through 2024.

Figure 5-1. Adjusted regulated against unregulated flow, by duration.

## 5.3 Synthetic Flood Ensembles

Only one water year in the record exceeds the unregulated 100-year peak, so the part of the unregulated-regulated relationship that sets the 100-, 250-, and 500-year regulated flows is essentially unconstrained by observed events. Synthetic floods are created by scaling the largest 12 events ranked by their unregulated flow estimate at Castle Rock and scaled to target the 100-, 250-, and 500-year unregulated peaks, plus a fourth that is 20 percent above the 500-year, resulting in 48 synthetic events of varying shapes and sizes.

Each source storm is scaled to hit its target unregulated peak and 5-day volume (within 5 percent), with Mossyrock inflow and Castle Rock local scaled by the same factor to preserve the observed coincidence. The events span regulated peaks of 80,884–266,744 cfs and attenuation ratios of 0.46–1.00.

Figure 5-2. Source storms and the scaled family built from each. The shaded band is the window over which the 5-day volume is matched.

## 5.4 The Unregulated-Regulated Transform

The regulated frequency curve is drawn through the observed and synthetic regulated flows with a locally weighted regression. The adjusted historical pairs (observed range) and the routed synthetic members (range above it) together give the (unregulated, regulated) pairs the transform is drawn through a locally weighted regression. The curve is constrained to increase monotonically and clipped at the 1:1 line since a regulated peak during a large event exceeding the regulation goal at Castle Rock would not exceed the unregulated peak.

Figure 5-3. Unregulated against regulated peak at Castle Rock.

## 5.5 Regulated Frequency Curve

Each ordinate of the adopted unregulated curve is read through the transform, inheriting its AEP. The regulated flow at a given AEP is what the projects produce from the unregulated flow of that AEP (Table 5-2). Mossyrock reduces the peak about 37 percent near the 1–2 percent AEP. The reduction is smaller at more frequent flows, where events are already largely contained, and at rarer flows, where storage is exhausted and the project must release more to avoid overtopping diminishing to about 26 percent at 0.1 percent AEP.

| Event | AEP | Unregulated (cfs) | Regulated (cfs) | Reduction | Regulated, 95% band (cfs) |
| --- | --- | --- | --- | --- | --- |
| 2-year | 0.500 | 60,529 | 50,813 | 16% | 37,799 – 62,798 |
| 10-year | 0.100 | 108,618 | 73,702 | 32% | 54,120 – 95,468 |
| 50-year | 0.020 | 153,556 | 96,784 | 37% | 62,569 – 158,023 |
| 100-year | 0.010 | 174,075 | 109,522 | 37% | 67,455 – 187,199 |
| 200-year | 0.005 | 195,977 | 128,798 | 34% | 76,214 – 228,500 |
| 500-year | 0.002 | 227,900 | 160,342 | 30% | 91,937 – 292,518 |
| 1000-year | 0.001 | 255,074 | 189,434 | 26% | 107,941 – 352,665 |

Table 5-2. Adopted regulated frequency curve at Castle Rock.

Figure 5-4. Adopted regulated and unregulated frequency curves at Castle Rock.

## 5.6 Uncertainty

The regulated flow at a given AEP carries uncertainty from two independent sources, the unregulated frequency estimate itself, and the scatter of the transform about its fitted line. Uncertainty from the unregulated frequency analysis is carried through to the regulated curve following EM 1110-2-1619 (UACE, 2025). The uncertainty band is treated as Normal on the upper and lower side of its best estimate separately which preserves the real asymmetry in both the HEC-SSP confidence limits.

The adopted band at the 95 percent two-sided level (z = 1.960):

Upper = RegBest + √[(Unreg97.5 − Unreg_best)² + (Transform97.5 − Transform_best)²]

Lower = RegBest − √[(Unreg_best − Unreg2.5)² + (Transform_best − Transform2.5)²]

The regulated upper bound is held at or below the unregulated upper bound above 60,000 cfs, since a regulated flood cannot exceed the unregulated flood it was routed from when Castle Rock is above its 50,000–70,000 cfs regulation goal. Below that threshold the project may release more than inflow to regain flood storage.

# 6. Comparison with the 2009 Restudy

## 6.1 Unregulated Curves

Figure 6-1 places the adopted unregulated curves against the 2009 Cowlitz Hydrology Restudy. The 2009 study adhered to Bulletin 17B and a period of record spanning most of 1927-2009 whereas the current study used Bulleting 17C, added 17 years of peaks to the record, and benefited from an updated regional skew study. The two studies agree within about 3 percent at the median and diverge at the tails. The 2009 adopted positive skews (0.140–0.193) that bend its curves upward in the tail, whereas the adopted skews here are near zero or negative.

Figure 6-1. Adopted 2026 unregulated curves (solid) against the 2009 Cowlitz Hydrology Restudy (dashed).

## 6.2 Regulated Curve

Figure 6-2 places the adopted regulated curve against the 2009 Restudy. The most notable divergence occurs beyond the 0.2 percent AEP when the 2009 regulated curve sharply approaches the unregulated curve. The 2009 study relied heavily on the 1933 event as the basis for synthetic storm development, which, as noted in section 5.5, has a shape that is likely to cause the reservoir to fill and spill. The current study scaled 12 events, including the 1933 event, to produce synthetics of varying shapes and found the 1933 event response to be atypical compared to other events with different shapes but similar peak and 5-day volumes that the reservoir was more successful in regulating. Uncertainty was not well defined for the 2009 regulated curve. Only one synthetic event was simulated per AEP at the upper tail and the curve was fit directly through those points and then hand drawn to converge with the unregulated curve.

Figure 6-2. Adopted 2026 regulated curve against the 2009 Cowlitz Hydrology Restudy.

Figure 6-3. Percent change between the 2026 and 2009 regulated curves.

# 7. Limitations

The transform is supported by data over the unregulated range of roughly 23,000–280,000 cfs. Its upper part rests entirely on synthetic events built by scaling observed storms, and hydrograph shape is not independent of size. A large storm is large because it was widespread and sustained, so scaling a moderate storm to a rare magnitude routes a shape that the storm never produced. Six of the twelve source storms are scaled past a factor of two to reach the largest target and should be read as the weaker evidence in the set.

The adjusted regulated record carries its own caveats: thirteen adjustments land within a narrow band near 14,000 cfs, more likely a fixed release constraint binding in one simulation and not the other than a pure starting-pool effect; seven adjustments exceed 40 percent of the observed peak, all moderate-peak years where a large absolute adjustment is a large fraction, influencing the lower end of the transform more than the upper.

# 8. Extension to the Columbia River Confluence

[TO BE COMPLETED]

The regulated frequency curve developed above applies at Castle Rock. Extending it downstream to the Columbia River confluence requires a coincident frequency analysis combining the regulated Cowlitz flow with Columbia River stage, supported by regional flood frequency estimates for the intervening ungaged area. That analysis is in progress and will be documented in this section, expected to cover the coincident frequency method and the assumed dependence between Cowlitz and Columbia flooding; the regional regression estimates for the ungaged local area below Castle Rock; the resulting regulated frequency estimates at the downstream locations required for levee analysis; and how the Section 5.6 uncertainty is carried through that combination.

# Appendix A. Assembled Unregulated Record by Water Year

Flows are in cubic feet per second. Source abbreviations: USGS peak, the observed instantaneous annual peak; USGS daily, rolling maxima of the observed daily record; Holdout, the hourly unregulated record from the Mossyrock holdout; Regression, the two-day storage-change relation; Mass balance, the daily unregulated series.

| WY | Peak | Peak source | 1-Day | 1-Day source | 3-Day | 5-Day | Duration source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1927 | 41,100 | USGS peak | — | — | — | — | — |
| 1928 | 74,000 | USGS peak | 71,300 | USGS daily | 56,933 | 47,940 | USGS daily |
| 1929 | 21,800 | USGS peak | 21,100 | USGS daily | 19,633 | 18,980 | USGS daily |
| 1930 | 30,200 | USGS peak | 29,500 | USGS daily | 26,700 | 23,840 | USGS daily |
| 1931 | 65,900 | USGS peak | 63,200 | USGS daily | 51,200 | 40,760 | USGS daily |
| 1932 | 50,800 | USGS peak | 49,200 | USGS daily | 44,467 | 39,140 | USGS daily |
| 1933 | 53,000 | USGS peak | 52,200 | USGS daily | 44,467 | 43,900 | USGS daily |
| 1934 | 139,000 | USGS peak | 134,000 | USGS daily | 120,333 | 106,300 | USGS daily |
| 1935 | 61,800 | USGS peak | 58,100 | USGS daily | 53,800 | 45,540 | USGS daily |
| 1936 | 48,200 | USGS peak | 39,800 | USGS daily | 35,833 | 31,400 | USGS daily |
| 1937 | 49,000 | USGS peak | 47,200 | USGS daily | 40,167 | 34,640 | USGS daily |
| 1938 | 62,900 | USGS peak | 60,500 | USGS daily | 53,567 | 47,440 | USGS daily |
| 1939 | 43,600 | USGS peak | 39,800 | USGS daily | 29,867 | 27,840 | USGS daily |
| 1940 | 41,900 | USGS peak | 40,500 | USGS daily | 36,767 | 31,660 | USGS daily |
| 1941 | 26,400 | USGS peak | 22,100 | USGS daily | 19,633 | 16,038 | USGS daily |
| 1942 | 58,100 | USGS peak | 55,400 | USGS daily | 47,067 | 39,400 | USGS daily |
| 1943 | 60,500 | USGS peak | 58,300 | USGS daily | 46,867 | 39,020 | USGS daily |
| 1944 | 37,100 | USGS peak | 33,700 | USGS daily | 26,467 | 20,760 | USGS daily |
| 1945 | 45,300 | USGS peak | 44,200 | USGS daily | 36,333 | 30,340 | USGS daily |
| 1946 | 54,700 | USGS peak | 51,700 | USGS daily | 45,433 | 38,940 | USGS daily |
| 1947 | 85,100 | USGS peak | 82,400 | USGS daily | 80,767 | 74,880 | USGS daily |
| 1948 | 38,500 | USGS peak | 37,000 | USGS daily | 32,700 | 28,560 | USGS daily |
| 1949 | 51,500 | USGS peak | 43,300 | USGS daily | 30,167 | 28,580 | USGS daily |
| 1950 | 54,800 | USGS peak | 54,000 | USGS daily | 49,933 | 43,920 | USGS daily |
| 1951 | 67,200 | USGS peak | 63,900 | USGS daily | 61,467 | 54,440 | USGS daily |
| 1952 | 41,400 | USGS peak | 35,600 | USGS daily | 31,600 | 28,680 | USGS daily |
| 1953 | 60,000 | USGS peak | 50,000 | USGS daily | 44,000 | 40,400 | USGS daily |
| 1954 | 73,000 | USGS peak | 63,600 | USGS daily | 51,067 | 45,440 | USGS daily |
| 1955 | 50,100 | USGS peak | 43,300 | USGS daily | 33,800 | 29,020 | USGS daily |
| 1956 | 67,400 | USGS peak | 63,100 | USGS daily | 49,733 | 42,260 | USGS daily |
| 1957 | 43,200 | USGS peak | 40,400 | USGS daily | 37,400 | 34,980 | USGS daily |
| 1958 | 35,200 | USGS peak | 34,500 | USGS daily | 30,867 | 27,280 | USGS daily |
| 1959 | 57,800 | USGS peak | 52,900 | USGS daily | 44,733 | 39,140 | USGS daily |
| 1960 | 62,700 | USGS peak | 61,000 | USGS daily | 56,433 | 49,620 | USGS daily |
| 1961 | 64,500 | USGS peak | 59,000 | USGS daily | 46,133 | 43,520 | USGS daily |
| 1962 | 42,100 | USGS peak | 40,100 | USGS daily | 34,833 | 30,240 | USGS daily |
| 1963 | 66,900 | USGS peak | 59,500 | USGS daily | 52,900 | 41,640 | USGS daily |
| 1964 | 72,600 | USGS peak | 55,400 | USGS daily | 46,267 | 37,680 | USGS daily |
| 1965 | 83,900 | USGS peak | 79,100 | USGS daily | 68,933 | 60,120 | USGS daily |
| 1966 | 28,800 | USGS peak | 27,000 | USGS daily | 24,333 | 22,400 | USGS daily |
| 1967 | 65,000 | USGS peak | 58,800 | USGS daily | 48,167 | 38,520 | USGS daily |
| 1968 | 49,500 | USGS peak | 47,600 | USGS daily | 44,633 | 42,600 | USGS daily |
| 1974 | 112,486 | Regression | 112,486 | Mass balance | 92,392 | 74,846 | Mass balance |
| 1975 | 76,896 | Regression | 71,250 | Mass balance | 54,411 | 50,666 | Mass balance |
| 1976 | 124,946 | Regression | 110,190 | Mass balance | 106,676 | 91,422 | Mass balance |
| 1977 | 19,899 | Regression | 16,299 | Mass balance | 14,752 | 14,436 | Mass balance |
| 1978 | 131,885 | Regression | 110,965 | Mass balance | 94,793 | 74,569 | Mass balance |
| 1979 | 51,587 | Regression | 40,506 | Mass balance | 28,000 | 25,107 | Mass balance |
| 1980 | 101,895 | Regression | 42,737 | Mass balance | 37,116 | 32,514 | Mass balance |
| 1981 | 88,345 | Regression | 86,532 | Mass balance | 71,889 | 57,905 | Mass balance |
| 1982 | 84,883 | Regression | 77,900 | Mass balance | 62,871 | 60,342 | Mass balance |
| 1983 | 82,466 | Regression | 72,298 | Mass balance | 55,966 | 52,485 | Mass balance |
| 1984 | 57,352 | Regression | 57,352 | Mass balance | 46,702 | 38,367 | Mass balance |
| 1985 | 33,098 | Regression | 33,098 | Mass balance | 28,765 | 23,969 | Mass balance |
| 1986 | 95,135 | Regression | 83,652 | Mass balance | 62,163 | 52,287 | Mass balance |
| 1987 | 103,298 | Regression | 90,982 | Mass balance | 61,617 | 50,210 | Mass balance |
| 1988 | 61,876 | Regression | 59,525 | Mass balance | 39,029 | 30,728 | Mass balance |
| 1989 | 28,884 | Regression | 23,674 | Mass balance | 21,120 | 19,496 | Mass balance |
| 1990 | 124,824 | Regression | 95,400 | Mass balance | 76,113 | 59,391 | Mass balance |
| 1991 | 110,107 | Regression | 104,020 | Mass balance | 75,569 | 58,070 | Mass balance |
| 1992 | 51,845 | Holdout | 43,323 | Holdout | 38,240 | 35,850 | Mass balance |
| 1993 | 33,272 | Regression | 31,533 | Mass balance | 26,576 | 21,802 | Mass balance |
| 1994 | 26,300 | Regression | 26,300 | Mass balance | 24,710 | 22,556 | Mass balance |
| 1995 | 64,114 | Regression | 59,736 | Mass balance | 48,317 | 39,125 | Mass balance |
| 1996 | 176,855 | Regression | 170,280 | Mass balance | 139,807 | 108,126 | Mass balance |
| 1997 | 86,921 | Holdout | 83,113 | Holdout | 74,563 | 69,706 | Mass balance |
| 1998 | 38,925 | Regression | 30,823 | Mass balance | 23,581 | 22,965 | Mass balance |
| 1999 | 76,193 | Holdout | 68,206 | Holdout | 60,807 | 51,702 | Mass balance |
| 2000 | 73,070 | Regression | 72,890 | Mass balance | 58,190 | 43,549 | Mass balance |
| 2001 | 19,568 | Regression | 19,568 | Mass balance | 15,339 | 14,332 | Mass balance |
| 2002 | 66,027 | Regression | 64,512 | Mass balance | 49,168 | 40,567 | Mass balance |
| 2003 | 109,719 | Holdout | 101,254 | Holdout | 72,896 | 53,892 | Mass balance |
| 2004 | 56,223 | Holdout | 52,144 | Holdout | 43,452 | 34,949 | Mass balance |
| 2005 | 52,859 | Regression | 50,133 | Mass balance | 39,349 | 31,339 | Mass balance |
| 2006 | 74,082 | Regression | 74,082 | Mass balance | 62,808 | 53,945 | Mass balance |
| 2007 | 147,777 | Regression | 147,777 | Mass balance | 100,470 | 73,836 | Mass balance |
| 2008 | 72,455 | Holdout | 70,265 | Holdout | 54,504 | 41,446 | Mass balance |
| 2009 | 151,296 | Holdout | 139,073 | Holdout | 99,324 | 72,961 | Mass balance |
| 2010 | 27,802 | Holdout | 25,172 | Holdout | 24,888 | 23,204 | Mass balance |
| 2011 | 87,160 | Holdout | 85,731 | Holdout | 71,689 | 58,118 | Mass balance |
| 2012 | 57,286 | Holdout | 53,334 | Holdout | 40,865 | 34,419 | Mass balance |
| 2013 | 58,409 | Regression | 47,950 | Mass balance | 36,797 | 30,359 | Mass balance |
| 2014 | 60,268 | Holdout | 57,504 | Holdout | 46,396 | 46,396 | Mass balance |
| 2015 | 47,323 | Holdout | 43,517 | Holdout | 32,555 | 31,664 | Mass balance |
| 2016 | 135,790 | Holdout | 124,896 | Holdout | 92,957 | 72,910 | Mass balance |
| 2017 | 64,342 | Holdout | 62,516 | Holdout | 51,010 | 47,186 | Mass balance |
| 2018 | 54,188 | Holdout | 52,984 | Holdout | 47,285 | 39,300 | Mass balance |
| 2019 | 24,389 | Holdout | 24,389 | Holdout | 24,389 | 22,197 | Mass balance |
| 2020 | 77,812 | Holdout | 73,544 | Holdout | 60,465 | 51,297 | Mass balance |
| 2021 | 61,703 | Holdout | 56,924 | Holdout | 42,829 | 35,700 | Mass balance |
| 2022 | 77,833 | Holdout | 73,803 | Holdout | 60,851 | 50,050 | Mass balance |
| 2023 | 65,623 | Holdout | 61,507 | Holdout | 47,710 | 35,341 | Mass balance |
| 2024 | 81,192 | Holdout | 76,898 | Holdout | 59,644 | 48,589 | Mass balance |
| 2025 | 45,954 | Holdout | 41,975 | Holdout | 35,253 | 30,112 | Mass balance |
| 2026 | 101,833 | Holdout | 97,675 | Holdout | 94,669 | 77,439 | Mass balance |

Table A-1. Assembled unregulated record, water years 1927 through 2026.

# Appendix B. Unregulated Frequency Curve Ordinates

Computed curve, expected probability curve, variance of the log, and 5 and 95 percent confidence limits for every ordinate reported by HEC-SSP. Flows are in cubic feet per second.

| Duration | AEP | Variance of log | z | Computed | 5% limit | 95% limit | Expected probability |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Peak | 0.0001 | 0.0098 | 3.72 | 296,088 | 476,290 | 217,104 | 374,643 |
| Peak | 0.0002 | 0.0082 | 3.54 | 276,072 | 426,543 | 207,624 | 332,739 |
| Peak | 0.0005 | 0.0064 | 3.29 | 250,134 | 366,728 | 194,487 | 285,568 |
| Peak | 0.001 | 0.0052 | 3.09 | 230,884 | 325,578 | 184,044 | 255,074 |
| Peak | 0.002 | 0.0041 | 2.88 | 211,932 | 287,635 | 173,114 | 227,900 |
| Peak | 0.005 | 0.0029 | 2.58 | 187,282 | 241,929 | 157,818 | 195,977 |
| Peak | 0.01 | 0.0022 | 2.33 | 168,884 | 210,386 | 145,504 | 174,075 |
| Peak | 0.02 | 0.0016 | 2.05 | 150,625 | 181,177 | 132,434 | 153,556 |
| Peak | 0.05 | 0.0010 | 1.64 | 126,514 | 145,732 | 113,770 | 127,758 |
| Peak | 0.1 | 0.0007 | 1.28 | 108,041 | 121,015 | 98,383 | 108,618 |
| Peak | 0.2 | 0.0005 | 0.84 | 88,923 | 97,624 | 81,626 | 89,126 |
| Peak | 0.5 | 0.0005 | 0.00 | 60,585 | 65,857 | 55,691 | 60,529 |
| Peak | 0.8 | 0.0006 | -0.84 | 40,671 | 44,531 | 36,709 | 40,487 |
| Peak | 0.9 | 0.0009 | -1.28 | 32,826 | 36,401 | 28,836 | 32,573 |
| Peak | 0.95 | 0.0013 | -1.64 | 27,417 | 30,925 | 23,227 | 27,041 |
| Peak | 0.99 | 0.0030 | -2.33 | 19,408 | 23,026 | 14,842 | 18,564 |
| 1-Day | 0.0001 | 0.0109 | 3.72 | 299,447 | 479,405 | 221,040 | 386,105 |
| 1-Day | 0.0002 | 0.0092 | 3.54 | 276,822 | 426,187 | 209,249 | 338,896 |
| 1-Day | 0.0005 | 0.0072 | 3.29 | 247,988 | 362,832 | 193,367 | 286,800 |
| 1-Day | 0.001 | 0.0059 | 3.09 | 226,952 | 319,700 | 181,106 | 253,245 |
| 1-Day | 0.002 | 0.0047 | 2.88 | 206,550 | 280,306 | 168,605 | 223,780 |
| 1-Day | 0.005 | 0.0034 | 2.58 | 180,481 | 233,404 | 151,652 | 189,945 |
| 1-Day | 0.01 | 0.0026 | 2.33 | 161,378 | 201,443 | 138,445 | 167,060 |
| 1-Day | 0.02 | 0.0019 | 2.05 | 142,727 | 172,189 | 124,837 | 145,949 |
| 1-Day | 0.05 | 0.0012 | 1.64 | 118,581 | 137,189 | 106,070 | 119,937 |
| 1-Day | 0.1 | 0.0009 | 1.28 | 100,464 | 113,080 | 91,102 | 101,103 |
| 1-Day | 0.2 | 0.0006 | 0.84 | 82,073 | 90,457 | 75,165 | 82,324 |
| 1-Day | 0.5 | 0.0005 | 0.00 | 55,503 | 60,349 | 51,025 | 55,470 |
| 1-Day | 0.8 | 0.0006 | -0.84 | 37,319 | 40,830 | 33,739 | 37,176 |
| 1-Day | 0.9 | 0.0008 | -1.28 | 30,257 | 33,490 | 26,713 | 30,053 |
| 1-Day | 0.95 | 0.0011 | -1.64 | 25,414 | 28,563 | 21,767 | 25,130 |
| 1-Day | 0.99 | 0.0022 | -2.33 | 18,268 | 21,476 | 14,392 | 17,638 |
| 3-Day | 0.0001 | 0.0093 | 3.72 | 240,363 | 376,364 | 180,427 | 298,915 |
| 3-Day | 0.0002 | 0.0079 | 3.54 | 222,081 | 335,087 | 170,391 | 264,436 |
| 3-Day | 0.0005 | 0.0062 | 3.29 | 198,868 | 285,924 | 157,011 | 225,645 |
| 3-Day | 0.001 | 0.0051 | 3.09 | 181,994 | 252,436 | 146,786 | 200,289 |
| 3-Day | 0.002 | 0.0041 | 2.88 | 165,677 | 221,830 | 136,451 | 177,857 |
| 3-Day | 0.005 | 0.0030 | 2.58 | 144,898 | 185,363 | 122,577 | 151,689 |
| 3-Day | 0.01 | 0.0023 | 2.33 | 129,717 | 160,485 | 111,876 | 133,835 |
| 3-Day | 0.02 | 0.0017 | 2.05 | 114,932 | 137,687 | 100,941 | 117,352 |
| 3-Day | 0.05 | 0.0011 | 1.64 | 95,836 | 110,351 | 85,995 | 96,884 |
| 3-Day | 0.1 | 0.0008 | 1.28 | 81,533 | 91,456 | 74,157 | 82,033 |
| 3-Day | 0.2 | 0.0006 | 0.84 | 67,026 | 73,645 | 61,589 | 67,236 |
| 3-Day | 0.5 | 0.0004 | 0.00 | 46,042 | 49,861 | 42,513 | 46,036 |
| 3-Day | 0.8 | 0.0005 | -0.84 | 31,599 | 34,399 | 28,743 | 31,493 |
| 3-Day | 0.9 | 0.0007 | -1.28 | 25,945 | 28,543 | 23,109 | 25,784 |
| 3-Day | 0.95 | 0.0010 | -1.64 | 22,044 | 24,587 | 19,120 | 21,810 |
| 3-Day | 0.99 | 0.0020 | -2.33 | 16,230 | 18,847 | 13,097 | 15,706 |
| 5-Day | 0.0001 | 0.0085 | 3.72 | 181,018 | 276,627 | 138,058 | 225,377 |
| 5-Day | 0.0002 | 0.0072 | 3.54 | 168,477 | 248,520 | 131,223 | 200,703 |
| 5-Day | 0.0005 | 0.0057 | 3.29 | 152,387 | 214,676 | 122,003 | 172,807 |
| 5-Day | 0.001 | 0.0046 | 3.09 | 140,566 | 191,358 | 114,869 | 154,542 |
| 5-Day | 0.002 | 0.0037 | 2.88 | 129,023 | 169,831 | 107,577 | 138,364 |
| 5-Day | 0.005 | 0.0027 | 2.58 | 114,149 | 143,861 | 97,651 | 119,354 |
| 5-Day | 0.01 | 0.0020 | 2.33 | 103,144 | 125,913 | 89,882 | 106,301 |
| 5-Day | 0.02 | 0.0015 | 2.05 | 92,301 | 109,270 | 81,834 | 94,152 |
| 5-Day | 0.05 | 0.0010 | 1.64 | 78,089 | 89,024 | 70,643 | 78,888 |
| 5-Day | 0.1 | 0.0007 | 1.28 | 67,267 | 74,814 | 61,613 | 67,651 |
| 5-Day | 0.2 | 0.0005 | 0.84 | 56,108 | 61,221 | 51,864 | 56,269 |
| 5-Day | 0.5 | 0.0004 | 0.00 | 39,563 | 42,625 | 36,714 | 39,551 |
| 5-Day | 0.8 | 0.0005 | -0.84 | 27,812 | 30,120 | 25,439 | 27,720 |
| 5-Day | 0.9 | 0.0006 | -1.28 | 23,104 | 25,274 | 20,704 | 22,966 |
| 5-Day | 0.95 | 0.0008 | -1.64 | 19,810 | 21,961 | 17,294 | 19,612 |
| 5-Day | 0.99 | 0.0018 | -2.33 | 14,823 | 17,089 | 12,041 | 14,367 |

Table B-1. Frequency curve ordinates, all four durations.

# Appendix C. Unregulated Frequency Curves by Duration

Individual curves as produced by HEC-SSP: computed curve, expected probability curve, 5 and 95 percent confidence limits, and observed events at Hirsch-Stedinger plotting positions. The peak figure reflects the censored treatment described in Section 4.8.

Figure C-1. Unregulated instantaneous peak frequency curve.

Figure C-2. Unregulated 1-day frequency curve.

Figure C-3. Unregulated 3-day frequency curve.

Figure C-4. Unregulated 5-day frequency curve.

# Appendix D. Adjusted Regulated Peak Record by Water Year

Observed, simulated and adjusted regulated peaks for every shared water year, with the screening result recorded for each. Flows are in cubic feet per second.

| WY | Observed | WCM_RC | Obs_RC | Difference | Adjusted | Screen |
| --- | --- | --- | --- | --- | --- | --- |
| 1974 | 68,600 | 33,207 | 19,208 | 13,999 | 82,599 | reg_over_unreg |
| 1975 | 58,900 | 57,075 | 56,714 | 361 | 59,261 | ok |
| 1976 | 86,700 | 75,597 | 105,150 | -29,553 | 86,700 | ok |
| 1977 | 15,000 | 14,187 | 12,187 | 2,000 | 17,000 | ok |
| 1978 | 86,400 | 79,778 | 79,778 | 0 | 86,400 | ok |
| 1979 | 39,400 | 42,972 | 40,972 | 2,000 | 41,400 | ok |
| 1980 | 97,000 | — | — | — | 97,000 | both |
| 1981 | 46,900 | 50,909 | 39,206 | 11,703 | 58,603 | ok |
| 1982 | 65,500 | 64,483 | 63,822 | 661 | 66,161 | ok |
| 1983 | 67,300 | 59,457 | 59,457 | 0 | 67,300 | ok |
| 1984 | 49,600 | — | — | — | 49,600 | different_event |
| 1985 | 23,600 | 24,934 | 13,724 | 11,210 | 23,600 | different_event |
| 1986 | 62,700 | 62,206 | 60,657 | 1,549 | 64,249 | ok |
| 1987 | 77,600 | 73,775 | 73,059 | 716 | 78,316 | ok |
| 1988 | 42,100 | 52,818 | 38,880 | 13,938 | 56,038 | ok |
| 1989 | 23,600 | 27,470 | 13,443 | 14,026 | 37,626 | ok |
| 1990 | 89,200 | 84,639 | 83,907 | 732 | 89,932 | ok |
| 1991 | 76,000 | 62,368 | 61,083 | 1,286 | 77,286 | ok |
| 1992 | 27,000 | 35,102 | 21,102 | 14,000 | 41,000 | ok |
| 1993 | 14,700 | 25,147 | 11,148 | 14,000 | 28,700 | ok |
| 1994 | 17,300 | — | — | — | 17,300 | different_event |
| 1995 | 47,500 | — | — | — | 47,500 | different_event |
| 1996 | 112,000 | 132,172 | 108,967 | 23,205 | 135,205 | ok |
| 1997 | 56,700 | 54,439 | 52,636 | 1,803 | 58,503 | ok |
| 1998 | 30,800 | 28,784 | 28,784 | 0 | 30,800 | ok |
| 1999 | 57,600 | 53,768 | 50,442 | 3,326 | 60,926 | ok |
| 2000 | 41,700 | 49,782 | 35,782 | 14,000 | 55,700 | ok |
| 2001 | 11,600 | — | — | — | 11,600 | different_event |
| 2002 | 51,400 | 53,728 | 45,573 | 8,155 | 59,555 | ok |
| 2003 | 69,000 | 62,340 | 62,077 | 263 | 69,263 | ok |
| 2004 | 38,100 | 48,301 | 34,302 | 14,000 | 52,100 | ok |
| 2005 | 29,600 | 40,940 | 26,912 | 14,029 | 43,629 | ok |
| 2006 | 56,200 | — | — | — | 56,200 | different_event |
| 2007 | 77,300 | 94,857 | 70,844 | 24,013 | 101,313 | ok |
| 2008 | 64,300 | 61,129 | 60,053 | 1,076 | 65,376 | ok |
| 2009 | 106,000 | 81,855 | 81,470 | 385 | 106,385 | ok |
| 2010 | 20,900 | 26,302 | 15,335 | 10,966 | 31,866 | ok |
| 2011 | 45,300 | 44,599 | 30,599 | 14,000 | 59,300 | ok |
| 2012 | 36,900 | 38,920 | 24,921 | 13,999 | 50,899 | ok |
| 2013 | 47,000 | 51,961 | 37,704 | 14,257 | 61,257 | reg_over_unreg |
| 2014 | 32,300 | 43,271 | 29,271 | 14,000 | 46,300 | ok |
| 2015 | 38,500 | 45,556 | 31,552 | 14,004 | 52,504 | ok |
| 2016 | 83,700 | 81,000 | 80,324 | 675 | 84,375 | ok |
| 2017 | 40,000 | 51,716 | 39,201 | 12,515 | 52,515 | ok |
| 2018 | 35,100 | 45,514 | 31,518 | 13,996 | 49,096 | ok |
| 2019 | 26,000 | — | — | — | 26,000 | different_event |
| 2020 | 47,400 | 51,580 | 51,580 | 0 | 47,400 | ok |
| 2021 | 45,600 | 50,963 | 42,650 | 8,313 | 53,913 | ok |
| 2022 | 63,900 | 62,387 | 61,747 | 639 | 64,539 | ok |
| 2023 | 34,400 | 44,819 | 30,826 | 13,993 | 48,393 | ok |
| 2024 | 44,800 | 52,638 | 42,979 | 9,659 | 54,459 | ok |

Table D-1. Adjusted regulated peak record, water years 1974 through 2024.

# Appendix E. Regulated Frequency Curve Ordinates and Uncertainty Terms

The adopted regulated curve with its uncertainty band (Section 5.6, FREQ_TERM_MODE = 'literal'), and the terms that produced it, for every ordinate. Flows are in cubic feet per second.

| AEP | Unregulated | Regulated | Lower 95% | Upper 95% | Local slope | Reduction % | Extrap. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.9900 | 18,564 | 18,564 | 11,620 | 24,732 | 1.00 | 0.0 | no |
| 0.9500 | 27,041 | 27,041 | 18,746 | 34,623 | 1.00 | 0.0 | no |
| 0.9000 | 32,573 | 32,573 | 23,312 | 41,155 | 1.00 | 0.0 | no |
| 0.8000 | 40,487 | 40,487 | 29,630 | 50,555 | 1.00 | 0.0 | no |
| 0.5000 | 60,529 | 50,813 | 37,799 | 62,798 | 0.57 | 16.1 | no |
| 0.2000 | 89,126 | 64,047 | 47,349 | 77,770 | 0.61 | 28.1 | no |
| 0.1000 | 108,618 | 73,702 | 54,120 | 95,468 | 0.73 | 32.1 | no |
| 0.0500 | 127,758 | 83,553 | 58,380 | 121,799 | 0.81 | 34.6 | no |
| 0.0200 | 153,556 | 96,784 | 62,569 | 158,023 | 0.86 | 37.0 | no |
| 0.0100 | 174,075 | 109,522 | 67,455 | 187,199 | 1.23 | 37.1 | no |
| 0.0050 | 195,977 | 128,798 | 76,214 | 228,500 | 1.43 | 34.3 | no |
| 0.0020 | 227,900 | 160,342 | 91,937 | 292,518 | 1.47 | 29.6 | no |
| 0.0010 | 255,074 | 189,434 | 107,941 | 352,665 | 1.49 | 25.7 | no |
| 0.0005 | 285,568 | 224,427 | 126,346 | 427,472 | 1.51 | 21.4 | yes |
| 0.0002 | 332,739 | 282,959 | 157,731 | 553,993 | 1.52 | 15.0 | yes |
| 0.0001 | 374,643 | 339,018 | 188,364 | 660,115 | 1.53 | 9.5 | yes |

Table E-1. Regulated frequency curve ordinates and uncertainty terms.

# References

England, J.F., Jr., Cohn, T.A., Faber, B.A., Stedinger, J.R., Thomas, W.O., Jr., Veilleux, A.G., Kiang, J.E., and Mason, R.R., Jr., 2019, Guidelines for determining flood flow frequency - Bulletin 17C (ver. 1.1): U.S. Geological Survey Techniques and Methods, book 4, chap. B5, 148 p.

Lind, G.D., Lamontagne, J.R., and Stonewall, A.J., 2020, Development of regional skew coefficients for selected flood durations in the Columbia River Basin, northwestern United States and British Columbia, Canada (ver. 1.1, October 2020): U.S. Geological Survey Scientific Investigations Report 2020-5073, 48 p.

Mastin, M.C., Konrad, C.P., Veilleux, A.G., and Tecca, A.E., 2016, Magnitude, frequency, and trends of floods at gaged and ungaged sites in Washington, based on data through water year 2014 (ver. 1.2, November 2017): U.S. Geological Survey Scientific Investigations Report 2016-5118, 70 p.

U.S. Army Corps of Engineers, 2025, Risk Assessment for Flood Risk Management Studies, EM 1110-2-1619, 29 September 2025.

U.S. Army Corps of Engineers, Portland District, 2009, Cowlitz River Hydrology Restudy.

U.S. Army Corps of Engineers, 2014, Water Control Manual, Mossyrock Project, Cowlitz River, Washington.

U.S. Army Corps of Engineers, Hydrologic Engineering Center, HEC-ResSim Reservoir System Simulation, and HEC-SSP Statistical Software Package.

U.S. Geological Survey, National Water Information System: streamflow and peak-flow records for station 14243000, Cowlitz River at Castle Rock, Washington.